3GPP TSG-SA3 Meeting #102-e 	S3-210077

# e-meeting, 18-29 January 20201

# Source:	NCSC

# Title:	Evaluation of solution 7

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is requested that SA3 approves this this evaluation of solution #7 for protecting the long term key when transporter along Nudr

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

blah.

# 4	Detailed proposal

START OF CHANGE 1

## 7.7	Solution #7: Transfer of the LTK out of the UDR

### 7.7.1	Introduction

This solution addresses key issue 3, "protection of long-term key during transfer out of UDR ".

The solution trusts the access tokens created using the OAuth 2.0 based authorization framework to protect the long-term key from retrieval by unauthorised NFs and to ensure it is only transported along the Nudr interface, the TLS protection on the Nudr interface, and that modification of the long-term key is restricted to the sequenceNumber attribute.

This solution is based on capabilities defined or planned to already to be defined in 3GPP and does not require any additional specification work.

### 7.7.2	Solution details

The OAuth 2.0 based authorization framework defined in 3GPP 33.501[2], clause 13.4.1, is enhanced in Release 16 to support the generation and validation of authorization tokens, including authorization at resource level. This allows the possibility of generating OAuth 2.0 access tokens to restrict retrieval of AuthenticationSubscription data to UDM/ARPF NF type service consumers only, for which the long-term key will be transported along the Nudr interface. As with any other SBA reference point, Nudr is protected at transport level using TLS as defined in 3GPP TS 33.501[2] clause 13.1.

As defined in 3GPP TS 29.505 [11], any modification of the long term key with the AuthenticationSubscription data resource by the UDM/ARPF over Nudr is limited to the sequenceNumber attribute, and attempts to modify any other attribute shall be rejected by the UDR.

### 7.7.3	Evaluation

This Solution proposes that the protection of the long-term key during transfer out of the UDR relies on the OAuth 2.0 based authorization framework defined in 3GPP TS 33.501 [2], and upon the restricted modification of the AuthenticationSubscription data type.

The capability for resource-based authorization introduced to the OAuth 2.0 authorization framework during release 16 ensures that the access to the authentication subscription data can be limited, and fulfils the security requirements to protect against unauthorized retrieval or modification of the long-term key while being transferred out of the UDR. The AuthenticationSubscription data type editing restrictions further protect against modification from any entity.

END OF CHANGE 1