3GPP TSG-SA3 Meeting #102-e 	S3-210078

# e-meeting, 18-29 January 20201

# Source:	NCSC

# Title:	Evaluation of solution 8

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is requested that SA3 approves this this evaluation of solution #8 for protecting OPc when transferred across Nudr

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution #8 for KI#5 is related to the protection of the Milenage OPc value during transfer out of the UDR over Nudr, and is based on same principle as the protection of the long-term key proposed by Solution #5 for KI#3, so the evaluation can take a similar form.

# 4	Detailed proposal

START OF CHANGE 1

## 7.8	Solution #8: Encrypted transfer of Milenage OPc value between UDR and UDM/ARPF

### 7.8.1	Introduction

This solution addresses key issue #5 on "protection of Milenage OPc value during transfer out of UDR".

If stored in the UDR, the Milenage OPc value is always transported in encrypted form during its transfer from UDR to UDM/ARPF.

### 7.8.2	Solution details

If stored in the UDR, the Milenage OPc value is transferred in encrypted form between UDR and UDM/ARPF. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

NOTE 1:	The selection of the encryption algorithm and the encryption/decryption key(s) is operator dependent.

The transfer of the encrypted Milenage OPc value over the Nudr interface is protected at transport level using the security mechanisms defined in 3GPP TS 33.501 [2], clause 13.1, as any other SBA interface.

During transfer, a key identifier is associated to the encrypted Milenage OPc value to enable identification of the decryption key in the UDM/ARPF.

NOTE 2:	The implementation of how to identify the decryption key is out of scope of 3GPP.

### 7.8.3	Evaluation

This solution addresses the requirements of the KI by protecting the transfer of the Milenage OPc value between the UDR and the UDM/ARPF in two ways:

-	transporting the Milenage OPc value in encrypted form during its transfer from UDR to UDM/APRF, and

-	additionally, protecting the transfer of the Milenage OPc value over Nudr based on secure encrypted transport mechanisms (such as HTTPS).

### This solution requires that the UDM/ARPF stores the decryption key. The storage of the decryption key at the UDM/ARPF is subject to the same security requirements as if the ARPF would store the Milenage OPc value. That is, the decryption key is required to be protected from physical attacks and never leave the secure environment of the UDM/ARPF unprotected. This required security of the decryption key can be achieved as it is done in pre-5G networks (e.g. by using a Hardware Security Module in the UDM/ARPF). It may be desirable to export a protected copy of the decryption key to a backup location, to aid recovery if necessary.

END OF CHANGE 1