3GPP TSG-SA3 Meeting #102-e 	S3-210080

# e-meeting, 18-29 January 20201

# Source:	NCSC

# Title:	Evaluation of solution 10

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is requested that SA3 approves this this evaluation of solution #10 for protecting OPc when stored in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

[2]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data".

# 3	Rationale

Solution #10 for KI#4 is related to the protection of the Milenage OPc value during storage in the UDR, and is based on same principle as the protection of the long-term key proposed by Solution #4 for KI#2, so the evaluation can take a similar form. For reference, TS 29.505 [2] defines the storage of the LTK and OPc in encrypted form in the UDR.

# 4	Detailed proposal

START OF CHANGE 1

## 7.10	Solution #10: Encrypted storage of Milenage OPc value in the UDR

### 7.10.1	Introduction

This solution addresses key issue #4 on "protection of Milenage OPc value during storage in UDR".

If the Milenage OPc value is stored in the UDR, it is stored in encrypted form in the UDR. During primary authentication, the UDM/ARPF retrieves authentication subscription data for the UE which can include the Milenage OPc value as stored in UDR. That is, the Milenage OPc value is never provided by the UDR in clear text and there is no need for the UDR to decrypt the Milenage OPc value.

At generation of the Milenage OPc value an encryption key is used that is shared with the UDM/ARPF where it is decrypted during primary authentication. The generation of the Milenage OPc value can be performed in the network of an operator or it can be performed at the facility where USIMs are being provisioned.

### 7.10.2	Solution details

If stored in the UDR, the Milenage OPc value is stored in encrypted form. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

NOTE:	The selection of the encryption algorithm and the encryption/decryption key(s) is operator dependent.

### 7.10.3	Evaluation

This solution addresses the requirements of the KI by protecting the OPc value in one way:

-	storing the OPc value in encrypted form in the UDR

This solution requires that the UDM/ARPF stores the decryption key. The storage of the decryption key at the UDM/ARPF is subject to the same security requirements as if the ARPF would store the long-term keys. That is, the decryption key is required to be protected from physical attacks and never leave the secure environment of the UDM/ARPF unprotected. This required security of the decryption key can be achieved as it is done in pre-5G networks (e.g. by using a Hardware Security Module in the UDM/ARPF). It may be desirable to export a protected copy of the decryption key to a backup location, to aid recovery if necessary.

END OF CHANGE 1