3GPP TSG-SA3 Meeting #102-e	S3-210159

# e-meeting, 18 - 29 January 2021, Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | S33.513 | CR | draftCR | rev | <Rev#> | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New security functional requirement and related test case on validating of user data transported over N9 by UPF with IPUPS functionality Title |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_IPUPS |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IPUPS can be deployed in UPF(s). Its functional requirement and related test case needs to be added to the TS 33.513. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new security functional requirment and related test case is added. Two references to TS 23.501(Release-16) and TS 33.501(Release-16) are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 2, 4.2.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************************start of change************************************************

#### 4.2.2.Z	Validating of user data transported over N9 by UPF with IPUPS functionality

Requirement Name: Validating of user data transported over N9 by UPF with IPUPS functionality.

Requirement Reference:

TS 23.501 [X], Clause 5.8.2.14; TS 33.501 [Y], Clause 5.9.3.4

Requirement Description:

"The IPUPS functionality forwards GTP-U packets (received via the N9 interface) only if they belong to an active PDU Session and are not malformed, as described in TS 33.501 [29]." as specified in TS 23.501[X], clause 5.8.2.14.

"The IPUPS shall only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others.

The IPUPS shall discard malformed GTP-U messages." as specified in TS 33.501[Y], clause 5.9.3.4.

Threat Reference: TR 33.926 [7], Clause L.2.W, "invalid user plane data forwarding"

TEST CASE:

Test Name: TC_UP_DATA_VALID_UPF_IPUPS

Purpose:

Verify that the UPF with IPUPS functionality only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all other.

Pre-Conditions:

Test environment is set up with UPF with IPUPS functionality under test. The tester is able to send GTP-U packets to the UPF with IPUPS functionality under test and receive GTP-U packets from the UPF with IPUPS functionality under test.

Execution Steps:

1)	The tester sends a GTP-U packet that contains an F-TEID that belongs to an active PDU session to the UPF with IPUPS functionality under test.

2)	The tester sends a GTP-U packet that doesn’t contain an F-TEID that belongs to an active PDU session to the UPF with IPUPS functionality under test.

3)	The tester receives GTP-U packet(s) from the UPF with IPUPS functionality.

Expected Results:

Only the GTP-U packet that contains an F-TEID that belongs to an active PDU session is received by the tester. The other one is not receirved by the tester.

Expected format of evidence:

Files containing the received GTP-U packets.

**************************************************next change************************************************

# 2	References

[X]	3GPP TS 23.501: "System Architecture for 5G System; Stage 2" (Release 16).

[Y]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

**************************************************end of change************************************************