3GPP TSG-SA3 Meeting #102-e 	S3-210168

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	ZTE Corporation

# Title:	Update evaluation of solution #4.4 in TR 33.846

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

This contribution proposes to update evaluation of solution #4.4.

2	References

(Reference - in list form - should be made to previous related SA3/3GPP/etc. documents.)

[1]    3GPP TR 33.846 “Study on authentication enhancements in 5G System”.

3	Rationale

This contribution proposes to update evaluation of solution #4.4.

4	Detailed proposal

**** START OF CHANGES ****

6.4.4.4	Evaluation

The proposed solution has impact on the USIM and UDM.

The solution prevents the attacker from retrieving any SEQMS information from UE in a way that the existing AK (Anonymity Key) used in AUTS is enough to protect SEQMS without the need for changes of the protocol messages or the cryptographic operations.

By setting AUTS to all zeros in case of RAND-reusage, a sniffer or active attacker on the radio interface does not get any information from the synchronization failure, except the fact that there was a RAND repetition. In case the RAND repetition is due to a false base station replaying an authentication request, which the network has previously sent to a UE, to this same UE, the attacker is anyway aware that this is a RAND repetition. In regular traffic, on the other hand, it is highly unlikely that two authentication requests that cause a synchronization failure will have the same RAND. If this still happens, and the network receives an all zeros AUTS, the network gets no proof that this AUTS really comes from the UE, and the network will not be able to synchronize the SEQHE with the SEQMS. In this case it is still clear for the network that something went wrong with the authentication procedure, so the authentication procedure needs to be repeated. The network repeats the authentication request until it creates a RAND that has not been used to compute an AUTS before.

A potential attack on recording RAND values could estimate the number of RAND values stored on the USIM by trial-and-error. Once an attacker has filled the RAND storage and can repeat previously sent RANDs, the solution no longer provides significantly more effective protection of the SQN than a USIM without such RAND storage. The difficulty of the attack can be increased by configuring the USIM with high RAND storage.

Editor's Note: The limitation of RAND storage in USIM is FFS.

The solution is not needed if time-based SQN generation is used.

**** END OF CHANGES ****