3GPP TSG-SA3 Meeting #102e 	S3-210201

# e-meeting, 18 - 29 January 2021													        Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	pCR – Address EN on alignment to SA2 procedure

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve this contribution to add a solution in TR33.854

# 2	References

[1]

# 3	Rationale

The contribution address the EN to align to SA2 procedure: sending “registration accept” at step 4.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

## 6.1	Solution #1: UAS Authentication and Authorization

### 6.1.1	Solution overview

This solution address the key issue #1.

This solution assumes each UAV or UAVC is provisioned with a PLMN UE ID (SUPI) and the corresponding credential so that it can be authenticated (primary authentication) by the PLMN as a normal UE. In addition, UAV or UAVC is provisioned with a UAS ID and corresponding credentials to perform UAS authentication and authorization (UAA) with USS/UTM.

The UAA is mandatory for UAA or UAVC and is based on EAP framework, where AMF is taking the role of the transparent Authenticator.

### 6.1.2	Solution details

The call flow of this solution is shown in the figure below.

Figure 6.1.2-1: UAA procedure

1. UAV (or UAVC) sends registration request to AMF. It may indicate that this is a registration for UAS.

NOTE: a new IE or an extension of an existing IE can be used to indicate UAA is requested. The IE can be defined in stage 3 and in coordination with CT.

2. AMF initiates Primary authentication as a normal UE

3. After successful Primary authentication, AMF checks whether UAV (or UAVC) requires UAA. This may be based on the subscription information retrieved from UDM in step 2

4. AMF returns a Registration Accept message to the UAV and indicates that UAA is pending.

5. UAA starts with EAP message exchanges.

a. 	AMF may optionally request UAS ID from UE.

b.	UAV (or UAVC) responses with UAS ID. It may indicate whether this is a UAV or UAVC.

c.	AMF sends UAA requests with UAS-ID and UAV or UAVC indicator in the EAP message. In addition, UAA request contains GPSI for USS/UTM to identify the UAV. GPSI shall be bound to UAS-ID.

d.	USS/UTM response with EAP messages accordingly

e.	EAP messages may continue based on the EAP method used.

f.	…

Note: the EAP authentication method used by UTM is out of scope of 3GPP

6. Based on the EAP authentication outcome, USS/UTM sends the results to AMF. If successful, USS/UTM sends the EAP-Success message, together with UAV/UAVC’s GPSI and UAS-ID that can uniquely identity the UAV/UAVC.

7. AMF stores the results, together with SUPI (converted from GPSI), UAS-ID, and UAV/UAVC indicator

8. AMF triggers the UE Configuration Update procedure . The message AMF sent to UE includes the UAS-ID and may include an indication it is for a UAV (or UAVC), if needed.

Editor's note:  Which core network function(s) (AMF, and/or others) and messaging will be used in the UAV authentication and authorization by USS/UTM procedure is FFS and in coordination with SA2

Editor's note:  How authorization revocation is supported should be marked as FFS

### 6.1.3	Solution evaluation

TBC

***	END OF CHANGES	***