3GPP TSG-SA3 Meeting #102e 	S3-210202

# e-meeting, 18 - 29 January 2021													        Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	pCR – Address EN on alignment to SA2 procedure

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve this contribution to add a solution in TR33.854

# 2	References

[1]

# 3	Rationale

To address the EN, this contribution introduces the “NFES” in the call flow.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

## 6.1	Solution #1: UAS Authentication and Authorization

### 6.1.1	Solution overview

This solution address the key issue #1.

This solution assumes each UAV or UAVC is provisioned with a PLMN UE ID (SUPI) and the corresponding credential so that it can be authenticated (primary authentication) by the PLMN as a normal UE. In addition, UAV or UAVC is provisioned with a UAS ID and corresponding credentials to perform UAS authentication and authorization (UAA) with USS/UTM.

The UAA is mandatory for UAA or UAVC and is based on EAP framework, where AMF is taking the role of the transparent Authenticator.

### 6.1.2	Solution details

The call flow of this solution is shown in the figure below.

Figure 6.1.2-1: UAA procedure

1. UAV (or UAVC) sends registration request to AMF. It may indicate that this is a registration for UAS.

NOTE: a new IE or an extension of an existing IE can be used to indicate UAA is requested. The IE can be defined in stage 3 and in coordination with CT.

2. AMF initiates Primary authentication as a normal UE

3. After successful Primary authentication, AMF checks whether UAV (or UAVC) requires UAA. This may be based on the subscription information retrieved from UDM in step 2

4. UAA starts with EAP message exchanges.

a. 	AMF may optionally request UAS ID from UE.

b.	UAV (or UAVC) responses with UAS ID. It may indicate whether this is a UAV or UAVC.

c.	AMF sends UAA requests to UFES (as defined in TR23.754) with UAS-ID and UAV or UAVC indicator in the EAP message. In addition, UAA request contains GPSI for USS/UTM to identify the UAV. GPSI shall be bound to UAS-ID. UFES locates the corresponding USS and forwards the UAA requests to it.

d.	USS/UTM response with EAP messages to AMF through UFES accordingly

e.	EAP messages may continue based on the EAP method used.

f.	…

Note: the EAP authentication method used by UTM is out of scope of 3GPP

5. Based on the EAP authentication outcome, USS/UTM sends the results to AMF through UFES. If successful, USS/UTM sends the EAP-Success message, together with UAV/UAVC’s GPSI and UAS-ID that can uniquely identity the UAV/UAVC.

6. AMF stores the results, together with SUPI (converted from GPSI), UAS-ID, and UAV/UAVC indicator

7. AMF sends UAS registration complete message to UE. The message includes the UAS-ID and may include an indication it is for a UAV (or UAVC), if needed.

Editor's note:  Whether the UUA steps are executed within or outside the Registration procedure is FFS and in coordination with SA2

Editor's note:  How authorization revocation is supported should be marked as FFS

### 6.1.3	Solution evaluation

TBC

***	END OF CHANGES	***