3GPP TSG-SA3 Meeting #102-e 	S3-210208

# e-meeting, 18 - 29 January 2021												            Revision of S3-21xxxx

# Source:	Huawei, HiSilicon

# Title:	Propose to add evaluation to Solution #17

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Approve this contribution to add evaluation to Solution#17 in TR 33.847

# 2	References

[1]	3GPP TS 33.847 Study on security aspects of enhancement for proximity based services in the 5G System (5GS).

# 3	Rationale

The contribution proposes to add evaluation of solution #17.

# 4	Detailed proposal

*************** BEGINNING OF CHANGES***************

## 6.17	Solution #17: Solution on securely creating destination Layer-2 ID in groupcast communication

### 6.17.1	Introduction

This solution addresses the KI #13 "Security and privacy of groupcast communication".

### 6.17.2	Solution details

The detailed solution is illustrated in Figure.

Figure 6.17.2-1: Procedure for secure conversion of application layer group ID to destination Layer-2 ID

0. Group Setup: Once the group is created, the group management server will send the application layer group ID to the associated UEs and a timer T. It will also send a set of random numbers and a specific sequence in which these random number are to be used. It is assumed that the application layer signalling is protected.

1.	ID Conversion: All the UEs use the application layer group ID and the first random number according to the sequence as an input to a hash function to generate the destination Layer-2 ID.

2.	ID Update: When the timer T expires, a new destination Layer-2 ID is calculated using the next random number according to the sequence. The UEs can listen to both old a new destination Layer-2 IDs, to avoid any time synchronization issues, for a certain period of time or until receives a message with the new ID.

The destination Layer-2 ID is updated until the ProSe application layer changes the group ID.

### The group management server can also send the corresponding materials to generate random numbers rather than sending the random numbers itself.

Editor’s Note: security for group management (including message 0 in the figure) is FFS

Editor’s Note: time synchronization among group members is FFS

### 6.17.3	Evaluation

The solution fulfils the first security requirement outlined by key issue #13. The solution provides a mechanism to securely convert the group ID to destination L2 ID as well as preserve the privacy of the destination L2 ID by regularly updating it.

*************** END OF CHANGES***************