3GPP TSG-SA3 Meeting #102-e 	S3-210222

# e-meeting, 18 - 29 January 2021

# Source:	China Unicom, Huawei, HiSilicon

# Title:	Check whether the N3IWF sends the EAP-Identity Request message

# Document for:	Approval

Agenda Item:	4.16

1	Decision/action requested

Approve this contribution to add a solution in TR33.520

2	References

[1]	3GPP TS 33.501, Security architecture and procedures for 5G System

3	Rationale

This contribution propose to add a new requirement for N3IWF.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

4.2.2.1.X	The capability to creat different child SAs

Requirement Name: The capability to Create Child SAs

Requirement Reference: TS 33.501 [1], clause 7.2.1

Requirement Description: "The N3IWF shall refrain from sending an EAP-Identity request. The UE may ignore an EAP Identity request or respond with the SUCI it sent in the Registration Request." as specified in TS 33.501 [1], clause 7.2.1.

Threat Reference:  TBD

Test Name: TC_N3IWF_REFRAIN_SEND_IDENTITY_REQUEST

Purpose:

Verify that the N3IWF will not send an EAP-Identity request message, and if the N3IWF sends EAP-Identity Request message, whether the N3IWF will conintue to proceed the EAP-5G procedure.

Pre-Conditions:

-	The AMF network product shall be connected in emulated/real network environments. UE may be simulated.

- 	A document describes whether the N3IWF will send an EAP-Identity Request message.

Execution Steps

1. The UE initiates IKE_SA_INIT procedure without UE identity.

2. The N3IWF responses an EAP message

Expected Results:

If the EAP message is EAP-5G start, then it is ok.

If the EAP message is EAP-Identity Request meesage, then check whether the N3IWF sends a N2 msg to the AMF, and whether the procedure in 7.2 of TS 33.501[1] runs successfully.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

***END OF CHANGES***