# 3GPP TSG-SA WG3 Meeting #102e 															  S3-210234

# e-meeting, 18 – 29 January 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | Draft CR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Threat analysis related to RRCConnectionReestablsihment in CP CIoT 5GS Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  |  | Date: |  | 2021-1-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If a UE experience a RLF when using Control Plane CIoT 5GS optimisation only, the AS layer of the UE may trigger an RRCConnectionReestablishment procedure. As there is no AS security available, this procedure is protected using NAS security, a NAS container is included in RRC Reestablishment Request message, and it will be sent by the ng-eNB to the AMF to do verification, the RAN does not know the vefification result, if the RAN receives Connection Establishment Indication message, then the RAN will trigger RRC Reestablishment message to the UE. Otherwise, if the RAN receives UE Context Release Command, the RAN will not send Reestablishment message to the UE. Thus, it should be ensured that the AMF can reply right result. Otherwise, it will cause waste of resources |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add threat analysis related to RRCConnectionReestablsihment in CP CIoT 5GS Optimization for AMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2.X.1 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***	BEGINNING OF CHANGES ***

K.2.X	Threat related to Security for 5G CIoT

K.2.X.1	Failed Verification of UE Identity during RRC Reestablishment Procedure for CP CIoT 5GS Optimization

-	Threat name: failed Verification of UE Identity during RRC Reestablishment Procedure for CP CIoT 5GS Optimization

-	Threat Category: Denial of Service.

-	Threat Description: If veritification of UE using CP CIoT 5GS Optimization during RRC Reestablishment procedure fails, a user identity cannot be verified. This can result in waste of system resources and deny a legitimate user access to the system. In addition, if the AMF does not correctly indicate the ng-eNB result of veritication, an unlegal UE may successfully re-establish on the ng-eNB, and result in waste of system resources.

-	Threatened Asset: Sufficient Processing Capacity.

***END OF CHANGES***