3GPP TSG-SA3 Meeting #102-e 	S3-21xxxx

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon, China Unicom

# Title:	Threat analysis on no new created child SA

# Document for:	Approval

Agenda Item:	4.16

1	Decision/action requested

Approve this contribution to add a solution in TR33.520

2	References

[1]

3	Rationale

Defined in TS 23.502, N3IWF shall determine the number of IPsec Child SAs to establish and the QoS profiles associated with each IPsec Child SA. For example, the N3IWF may decide to establish one IPsec Child SA and associate all QoS profiles with this IPsec Child SA. In this case, all QoS Flows of the PDU Session would be transferred over one IPsec Child SA. Therefore when a N3IWF cannot create a new child SA for different QoS, the user data will be sent in a channel that lead to a downgrade user experience or upgrade experience.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

Annex X (normative): 
Aspects specific to the network product class N3IWF

X.Y	No new created child SA

-	Threat name: Create a new child SA for a PDU session

-	Threat Category: Elevation of privilege.

-	Threat Description: The N3IWF may decide to establish one IPsec child SA and associate all QoS profiles with this IPsec child SA. In this case, all QoS Flows of the PDU Session would be transferred over one IPsec child SA. N3IWF may also decide to establish different child SAs corresponding to the different QoS flows. If the N3IWF loses the capability of creating different Child SA, the QoS requirement of data of this PDU session maybe downgraded or upgraded by using a different QoS Channel.

-	Threatened Asset: Sufficient processing capacity.

***END OF CHANGES***