3GPP TSG-SA3 Meeting #102-e 	S3-210270

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon, China Unicom

# Title:	Check whether the N3IWF has the ability to create different child SAs

# Document for:	Approval

Agenda Item:	4.16

1	Decision/action requested

Approve this contribution to add a solution in TR33.520

2	References

[1]

3	Rationale

This contribution propose to add a new requirement for N3IWF.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

***END OF 1st CHANGES***

***	BEGINNING OF 2nd CHANGES ***

4.2.2.1.X	The capability to creat different child SAs

Requirement Name: The capability to Create Child SAs

Requirement Reference: TS 23.502 [XX], clause 4.12.5.

Requirement Description: "N3IWF shall determine the number of IPsec Child SAs to establish and the QoS profiles associated with each IPsec Child SA. For example, the N3IWF may decide to establish one IPsec Child SA and associate all QoS profiles with this IPsec Child SA. In this case, all QoS Flows of the PDU Session would be transferred over one IPsec Child SA." as specified in TS 23.502 [XX], clause 4.12.5.

Threat Reference:  TBD

Test Name: TC_N3IWF_CREATE_DIFFERENT_CHILD_SA

Purpose:

Verify that the N3IWF has the capability to creat different child SAs.

Pre-Conditions:

-	The AMF network product shall be connected in emulated/real network environments. UE may be simulated.

- 	A document describes whether the N3IWF has the ability to creat different child SAs and the trigger condition.

Execution Steps

1. The UE sends Registration Request message over non-3GPP access to the AMF.

2. The AMF sends Registration Accept message to the UE.

3. The UE sends PDU Session Establishement Request message to the AMF.

4. The AMF send a N2 PDU Session Request message to the N3IWF to establish the access resources for this PDU session with a QoS in one condition.

5. Repeat step 3-4 with a differen QoS in another condition.

Expected Results:

The N3IWF creates more than one child SA.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

***END OF CHANGES***