3GPP TSG-SA3 Meeting #102-e 	S3-210271

# e-meeting, 18 - 29 January 2021, Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | Draft CR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Threat analysis on mixing CP and UP into main SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_N3IWF |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | R-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 6.8.1.2.3 of TS 33.501, it says “The main SA is used solely for the transport of NAS messages between the UE and the AMF/SMF.” Therefore, if the N3IWF mixed use main SA for CP and UP, it does not comply with the design principle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new threat |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No threat mapping to a test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***	BEGINNING OF CHANGES ***

Annex X (normative): 
Aspects specific to the network product class N3IWF

X.Y	Threat to mix CP and UP into main SA

-	Threat name: mixed use main SA

-	Threat Category: Elevation of privilege.

-	Threat Description: The N3IWF shall cerate at least one child SA for UP traffic, The child SA for UP traffic will reflect the QoS requirement, if the N3IWF fails to create a child SA, the user plane will reuse the control plane SA and this creats bidding down attack on the user plane on QoS parameter.

-	Threatened Asset: Sufficient processing capacity.

***END OF CHANGES***