GPP TSG-SA3 Meeting #102-e 	S3-210272

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	Check whether the N3IWF creates a child SA for PDU session

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve this contribution to add a solution in TR33.520

2	References

[1]

3	Rationale

This contribution propose to add a new requirement for N3IWF.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

***END OF 1st CHANGES***

***	BEGINNING OF 2nd CHANGES ***

4.2.2.1.X	Create a new Child SA for PDU session

Requirement Name: Create a new Child SA for a PDU session

Requirement Reference: TS 33.501 [XX], clause 6.8.1.2.3

Requirement Description: " The main SA is used solely for the transport of NAS messages between the UE and the AMF/SMF. " as specified in TS 33.501 [XX], clause 6.8.1.2.3.

Threat Reference:  TBD

Test Name: TC_N3IWF_CREATE_CHILD_SA

Purpose:

Verify that the N3IWF creates a new child SA for a PDU session.

Pre-Conditions:

-	The AMF network product shall be connected in emulated/real network environments. UE may be simulated.

Execution Steps

1. The UE sends Registration Request message over non-3GPP access to the AMF.

2. AMF Registration Accept message to the UE.

3. The UE sends PDU Session Establishment Request message to the AMF.

4. The AMF send a N2 PDU Session Request message to the N3IWF to establish the access resources for this PDU session.

Expected Results:

The N3IWF creates a child SA after step4.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

***END OF CHANGES***