3GPP TSG-SA3 Meeting #102-e 	S3-210273

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	Threat analysis on select AAA-P and AAA-S

# Document for:	Approval

Agenda Item:	4.18

1	Decision/action requested

Approve this contribution to add a solution in TR33.520

2	References

[1]

3	Rationale

AAA-S in NSSAA procedure may be hosted by the HPLMN or third party which has a business relationship. When AAA-S belongs to a third party, the AAA-P in the HPLMN may be involved. Different S-NSSAI may go to different AAA-S. If the NSSAF does not have the ability to route the S-NSSAI to the right place, the authentication will be always fail.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

Annex X (normative): 
Aspects specific to the network product class NSSAAF

X.Y	Threat to select AAA-P and AAA-S

-	Threat name: AAA-P and AAA-S wrong selection

-	Threat Category: Denial of service.

-	Threat Description: AAA-S in NSSAA procedure may be hosted by the HPLMN or third party which has a business relationship. When AAA-S belongs to a third party, the AAA-P in the HPLMN may be involved. Different S-NSSAI may go to different AAA-S. If the NSSAAF does not have the ability to select the right receiver, the authentication will always fail.

-	Threatened Asset: GNP Application.

***END OF CHANGES***