GPP TSG-SA3 Meeting #102-e 	S3-210274

# e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	NSSAAF routes the S-NSSAAI to the right place

# Document for:	Approval

Agenda Item:	4.18

1	Decision/action requested

Approve this contribution to add a solution in TR33.5xx

2	References

[1]

3	Rationale

This contribution propose to add a new requirement for NSSAAF SCAS.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

***END OF 1st CHANGES***

***	BEGINNING OF 2nd CHANGES ***

4.2.2.1.X	Routes the S-NSSAI to the right place

Requirement Name: Routes the S-NSSAI to the right place

Requirement Reference: TS 33.501 [XX], clause 6.8.1.2.3

Requirement Description: " If the AAA-P is present (e.g. because the AAA-S belongs to a third party and the operator deploys a proxy towards third parties), the NSSAAF forwards the EAP ID Response message to the AAA-P, otherwise the NSSAAF forwards the message directly to the AAA-S. NSSAAF routes to the AAA-S based on the S-NSSAI. " as specified in TS 33.501 [XX], clause 6.13.

Threat Reference:  TBD

Test Name: TC_NSSAAF_CORRECT_ROUTING

Purpose:

Verify that the NSSAAF forwards the NSSAA request to the right receiving end.

Pre-Conditions:

-	Test environment with AMF, AAA-S and AAA-P, which may be simulated. The NSAAF under test is connected with AMF, AAA-S and AAA-P.

- 	A document describes the logic how the NSSAAF selects an AAA-S or AAA-P based on S-NSSAI.

- 	Preconfigure the NSSAAF under test with two routing entries, each for a NSSAI. One of the slice is a part of MNO and the AAA-S can be directly found by the NSSAAF, while the other slice serves 3rd party and the AAA-P will be used for NSSAA procedure.

Execution Steps

1. The AMF sends Nssaaf_NSSAA_Authenticate Req to the NSSAAF including one of the S-NSSAI.

2. The NSSAAF sends AAA message to an AAA-P.

3. Repeat step 1 and 2 with the other S-NSSAI, and the NSSAAF sends AAA message to an AAA-S.

Expected Results:

The NSSAAF forwards the NSSAA request to the correct AAA-S or AAA-P on the S-NSSAI

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

***END OF CHANGES***