3GPP TSG-SA3 Meeting #102-e                   	    S3-210323

# e-meeting, 18 - 29 January 2021														Revision of S3-21xxxx

# Source:	China Mobile, Huawei, HiSilicon

# Title:	Find the right NF instance are serving the UE

# Document for:	Approval

Agenda Item:	4.15

1	Decision/action requested

Approve this contribution to add a solution in TS33.521

2	References

[1]

3	Rationale

This contribution propose to add a new requirement for NWDAF.

4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services".

***END OF 1st CHANGES***

***	BEGINNING OF 2nd CHANGES ***

4.2.X	Finding the right NF instance are serving the UE

Requirement Name: Finding the right NF instance are serving the UE

Requirement Reference: TS 23.288[XX], clause 6.2.2.1.

Requirement Description: "To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

." as specified in TS 23.288 [XX], clause 6.2.2.1.

Threat Reference:  TBD

Test Name: TC_NWDAF_FIND_NF_SRVING_UE

Purpose:

Verify that the NWDAF does not always find an NF from operations related to the UE..

Pre-Conditions:

TBD

Execution Steps

TBD

Expected Results:

TBD

Expected format of evidence:

TBD

***END OF CHANGES***