3GPP TSG-SA3 Meeting #102e	S3-210348

# e-meeting, 18 – 29 January 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | Draft CR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Threats related to session establishment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei;Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 18-01-2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | R-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | After the successful network slice-specific authentication and authorization, there will be an Allowed NSSAI list both in UE and AMF. Then, the UE will initiate the PDU session establishment procedure with requested S-NSSAIs included. However, if the AMF dose not check whether the requested S-NSSAIs are within its stored Allowed NSSAI list of the UE, the UE can still access the slice service even if it fails the NSSAA procedure by including the failed S-NSSAI in the PDU session establishment request. Therefore, it is proposed to analyse the potential threats when the AMF does not correctly validate the requested S-NSSAIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new threat analysis to 33.926 related to session establishment authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threat cannot be well identified and the test on it cannot find its threat reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Change 1************

K.2.x	Threats related to session establishment procedure

K.2.x.1	Incorrect validation of S-NSSAIs

-	Threat name: Incorrect Validation of S-NSSAIs.

-	Threat category: Elevation of Privilege.

-	Threat Description: After the successful network slice-specific authentication and authorization, there will be an Allowed NSSAI list both in UE and AMF. Then, the UE will initiate the PDU session establishment request with the requested S-NSSAIs included. If the AMF does not verify whether the received S-NSSAIs is within the Allowed NSSAI list stored at the AMF, an attacker can still include the rejected S-NSSAIs in the request and access the slice after it fails the NSSAA procedure.

-	Threatened Asset: Mobility management data, sufficient processing capacity.

************* End of Change 1************