3GPP TSG-SA3 Meeting #102-e 	S3-210355

# e-meeting, 18 -29 January 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.926 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assets and threats specific of encryption in network hiding in the I-CSCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_IMS |  |  |  |  |  | Date: |  |  | 2021-1-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The assets and threats related to encryption in network hiding in the I-CSCF needs to be added. In cases of the network hiding mechanism is used and the operator policy states that the topology shall be hidden, if the encryption of the hiding information elements is not performed when the I-CSCF forwards SIP Request or Response messages outside the hiding network’s domain, and the decryption of the hiding information elements is not performed when the I-CSCF receives a SIP Request or Response message from outside the hiding network’s domain, the identities of the SIP proxies and the topology of the hiding network will not be protected, and an attacker can read or modify these information elements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding assets and threats related to encryption in network hiding in the I-CSCF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No reference of assets and threats related to encryption in network hiding in the I-CSCF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex X.Y.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

X.Y	Assets and threats specific to the I-CSCF

X.Y.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the I-CSCF to be protected are:

-	I-CSCF Application

-	IMS signalling, the Address of the S-CSCF, Charging data records

-	Security data, i.e. cryptographic materials for Mw, Cx, Mx, Ma, and Mm interfaces

-	The interfaces of the I-CSCF to be protected and which are within SECAM scope:

-	Mw interface between the I-CSCF and S-CSCF/P-CSCF

-	Cx interface between the I-CSCF and the HSS and SLF

-	Mx interface between the I-CSCF and the IBCF

-	Ma interface between the I-CSCF and AS

-	Mm interface between the I-CSCF and IP Multimedia Networks

-	Console interface, for local access: local interface on the I-CSCF

-	OAM interface, for remote access: interface between the I-CSCF and the OAM system

NOTE 1: 	The detailed interfaces of the I-CSCF class are described in clause 4 of the present document.

-	I-CSCF Software: binary code or executable code

NOTE 2: 	I-CSCF files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, I-CSCF application, user plane security mechanism, or cryptographic materials.

X.Y.2	Threats related to network hiding

X.Y.2.1	encryption in network hiding

-	Threat name: Encryption in network hiding

-	Threat Category: Spoofing identity, Tampering of data, Information Disclosure

-	Threat Description: In casse of the network hiding mechanism is used and the operator policy states that the topology shall be hidden, if the encryption of the hiding information elements is not performed when the I-CSCF forwards SIP Request or Response messages outside the hiding network’s domain, and the decryption of the hiding information elements is not performed when the I-CSCF receives a SIP Request or Response message from the outside of the hiding network’s domain, the identities of the SIP proxies and the topology of the hiding network will not be protected, and an attacker can read or modify these information elements.

-	Threatened Asset: IMS signalling

*************** End of the Change ****************