3GPP TSG-SA3 Meeting #102-e 	S3-210357

# e-meeting, 18 -29 January 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.926 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assets and threats specific of network hiding in the IBCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_IMS |  |  |  |  |  | Date: |  |  | 2021-1-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The assets and threats related to network hiding in the IBCF needs to be added. If the network topology hiding is required, the encryption of the hiding information and the replacement of the hiding information can be used as the network hiding mechanism. If the encryption and the replacement of the hiding information elements is not performed when the IBCF forwards SIP Request or Response messages outside the hiding network’s domain, and the decryption and the replacement of the hiding information elements is not performed when the IBCF receives a SIP Request or Response message from outside the hiding network’s domain, the identities of the SIP proxies and the topology of the hiding network will not be protected, and an attacker can read or modify these information elements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding assets and threats related to network hiding in the IBCF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No reference of assets and threats related to network hiding in the IBCF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex X.Y.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

X.Y	Assets and threats specific to the IBCF

X.Y.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the IBCF to be protected are:

-	IBCF Application

-	IMS signalling, Network configuration hiding, Charging data records

-	Security data, i.e. cryptographic materials for Mx, Cs, Ix, and Ici interfaces

-	The interfaces of the IBCF to be protected and which are within SECAM scope:

-	Mx interface between the IBCF and S-CSCF/P-CSCF/I-CSCF/BGCF

-	Ms interface between the IBCF and the AS

-	Ix interface between the IBCF and the TrGW

-	Ici interface between the IBCF and IP Multimedia Networks

-	Console interface, for local access: local interface on the IBCF

-	OAM interface, for remote access: interface between the IBCF and the OAM system

NOTE 1: 	The detailed interfaces of the IBCF class are described in clause 4 of the present document.

-	IBCF Software: binary code or executable code

NOTE 2: 	IBCF files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, IBCF application, user plane security mechanism, or cryptographic materials.

X.Y.2	Threats related to network hiding

X.Y.2.1	encryption in network hiding

-	Threat name: Encryption in network hiding

-	Threat Category: Spoofing identity, Tampering of data, Information Disclosure

-	Threat Description: In cases of the encryption of the hiding information as network hiding mechanism is used and the operator policy states that the topology shall be hidden, and the encryption of the hiding information elements is not performed when the IBCF forwards SIP Request or Response messages outside the hiding network’s domain, and the decryption of the hiding information elements is not performed when the IBCF receives a SIP Request or Response message from the outside of the hiding network’s domain, the identities of the SIP proxies and the topology of the hiding network will not be protected, and an attacker can read or modify these information elements.

-	Threatened Asset: IMS signalling

X.Y.2.2	replacement in network hiding

-	Threat name: Replacement in network hiding

-	Threat Category: Spoofing identity, Tampering of data, Information Disclosure

-	Threat Description: In cases of the replacement of the hiding information as network hiding mechanism is used and the operator policy states that the topology shall be hidden, and the hiding information elements are not replaced to constant values when the IBCF forwards SIP Request or Response messages outside the hiding network’s domain, and the constant values are not replaced to the hiding information elements when the IBCF receives a SIP Request or Response message from the outside of the hiding network’s domain, the identities of the SIP proxies and the topology of the hiding network will not be protected, and an attacker can read or modify these information elements.

-	Threatened Asset: IMS signalling

*************** End of the Change ****************