3GPP TSG-SA3 Meeting #102-e 	S3-210377

# e-meeting, 18 – 29 January 2021, online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | DRAFT | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the Ens related with biding down attack in the threats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_IMS |  |  |  |  |  | Date: |  |  | 2020-12-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The only requirement defined in TS 33.203 in case the P-CSCF configured to apply confidentiality is that the P-CSCF always includes the encryption algorithms in SM6, which it supports. Since no detailed following handlings of the P-CSCF is captured in any places, it is assume that the following handlings of the P-CSCF can be left for implementation. Therefore, the EN related with the following analysis of the P-CSCF can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the Editor’s notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One Editor’s notes is unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.2.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st changes ****************

X.2.3	Threats related to IMS signalling transport

- Threat name:  No protection or weak protection for IMS signalling data.

- Threat Category: Tampering, Information Disclosure.

- Threat Description: The following behaviours may lead to bidding down attacks

-	If the protection implemented for the IMS signalling over Gm interface uses the wrong security profile, which may contain weak security algorithms or protocol versions known to be vulnerable, the level of the security of the IMS signalling data may be degraded and fail to fulfil the required security.

-	The P-CSCF may be configured to never apply confidentiality, because e.g. it trusts the encryption provided by the underlying access network. The P-CSCF may also be configured to apply confidentiality whenever the UE supports it. During security associateion set-up, the first message SM1 “Register” may not be protected, hence the information within SM1 could be tampered by an attacker, e.g. by removing the encryption algorithms in the “UE integrity and encryption algorithms list”. In such case, the P-CSCF will not receive the encryption algorithms supported by the UE and may mistakenly believe that the UE does not support any encryption algorithm, hence will select NULL algorithm for encryption. If the P-CSCF configured to apply confidentiality does not includes the encryption algorithms it supports in SM6 when receiving no supported encryption algorithms from the UE, the UE may mistakenly believe that the P-CSCF is configured to not apply confidentiality when receiving SM6 and will select NULL algorithm for encryption. Therefore, NULL encryption algorithm is negotiated between the UE and the P-CSCF and confidentiality will eventually not be provided for the security association, in which way the attacker can launch the bidding down attack. In another word, if the P-CSCF configured to apply confidentiality does not include its encryption algorithms in SM6 when receiving no encryption algorithms of the UE in SM1, such behavior of P-CSCF will fail to thwart the bidding down attack.

NOTE:	The threat above applies to UEs supporting at least one encryption algorithm other than NULL algo, as an attacker cannot launch such bidding down attack on UEs only supporting NULL algo.

- Threatened Asset: IMS signalling data.

*************** End of the 2nd changes ****************