3GPP TSG SA WG 3 Meeting SA3#102e		TDoc S3-210459

Electronic meeting, Online, 18 - 29 January 2021

Title:	Reply LS on Storage of KAUSF

Response to:	LS S3-210005/C1-207764 on Reply LS on Storage of KAUSF from CT1

Source:	SA3

To:	CT1

Cc:	---

Contact person:	Rajavelsamy Rajadurai

Rajvel@]Samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-210460.

# 1	Overall description

SA3 thanks CT1 for their LS on Storage of KAUSF (S3-210005/C1-207764).

In SA3#101e meeting, SA3 started the discussion on the issue, when the new KAUSF is stored at the UE as part of alignment with the CT4 specification for Rel-16. SA3 continued the discussion and agreed on the attached CR S3-210460 in the SA3#102e meeting, particularly changes to the clause 6.2.2.2 is relevant to this issue.

The guideline from SA3 is, only after identifying that the primary (re)authentication is successful in the network side, the UE shall store the KAUSF, KSEAF, SOR counter and UE parameter update counter on the USIM (or in the non-volatile memory of the ME, if no corresponding file is present on the USIM). The UE identifies that the primary (re)authentication is successful in the network side, upon receiving a valid integrity protected response message (other than Authentication Reject message) related to the NAS procedure as part of which the primary authentication was performed or if the UE moves to or remains in RM-REGISTERED state. Instead, if an Authentication Reject message is received or if the UE moves to or remains in RM-DEREGISTERED state, then the performed primary (re)authentication is considered as unsuccessful in the network side.

In abnormal cases, if the NAS procedure completes unsuccessfully (no response from the network/UE) and if a primary (re)authentication was also performed as part of the unsuccessful NAS procedure, then there is possibility for misalignment in the stored KAUSF in the UE and in the network. SA3 assumes that, in such abnormal cases i.e.,:

- if there is no response from the UE (for example, due to RLF), then to recover from a failure to align the KAUSF due to an unsuccessful NAS procedure, the SEAF shall initiate a new primary (re)authentication procedure during any subsequent procedure establishing a signalling connection with the UE, to align the KAUSF stored in the UE and in the network.

- if there is no response from the network (for example, no NAS response), then to recover from a failure to align the KAUSF due to an unsuccessful NAS procedure, the UE shall enter the state 5GMM-DEREGISTERED.INITIAL-REGISTRATION-NEEDED, as to perform the registration procedure for initial registration as soon as possible.

SA3 kindly request CT1 to check SA3 assumptions on the recovery procedures from the abnormal cases and provide feedback.

# 2	Actions

To CT1

ACTION: 	SA3 would like to kindly ask CT4 to take the above guidance into account. Further SA3 kindly request CT1 to check the SA3 assumption on the recovery procedure from the abnormal cases and provide feedback.

# 3	Dates of next TSG SA WG 3 meetings

SA3#102e-Bis	1 - 5 March 2021	Electronic meeting

SA3#103e	17 - 28 may 2021	Electronic meeting