3GPP TSG-SA3 Meeting #103-e 	S3-211710

# e-meeting, 17- 28 May 2021

# Source:	Apple

# Title:	Evaluation on solution#11

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is proposed to add the evaluation for solution#11 in MEC TR 33.839.

# 2	References

[1]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)"

# 3	Rationale

This pCR proposes to add the evaluation on solution#11.

# 4	Detailed proposal

****START OF CHANGES ***

### 6.11	Solution #11: Authentication between EEC and ECS

### 6.11.1	Solution overview

This solution addresses the security requirement for the Authentication between EEC and ECS in key issue #2.

In this solution, UE knows to use AKMA with ECS via interactions with 3GPP network before communication with ECS. If the ECS deployed by MNO is considered to be trusted by the operator, the ECS interacts directly with AAnF. Otherwise, the ECSs not allowed by the operator to access directly the Network Functions should use the NEF to interact with AAnF.

### 6.11.2	Solution details

Figure 6.11.2-1 Authentication between the EEC and ECS based AKMA

0a. UE performs primary authentication with the network. Then KAUSF is shared between UE and AUSF in Home network. If the AUSF receives the AKMA indication from the UDM, the AUSF should generate the AKMA Anchor Key (KAKMA) and the A-KID from KAUSF after the primary authentication procedure is successfully completed.

0b. After AKMA key material is generated, the AUSF should send the generated A-KID, and KAKMA to the AAnF.

- During PDU Session Establishment, the SMF provides Edge Configuration Server Information (ECS ID (i.e. FQDN or IP address(es) of ECS), AKMA capability) to the UE. The AKMA capability indicates the ECS support to use AKMA.

- When the UE determines to communicate with ECS, if the UE supports AKMA, the UE derives the AKMA key and KECS(i.e KAF) as specified in TS 33.535 [6] based on the received AKMA capability.

- UE computes the MAC-I over the request message using the KECS and sends service provision request with A-KID, MAC-I.

NOTE:	TLS based on AKMA PSK solution is defined in other solutions.

- Upon receiving the request, the Edge Configuration Server discovers the AAnF or NEF.

NOTE :	In the case of architecture without CAPIF support, the ECS is locally configured with the API termination points for the service. In the case of architecture with CAPIF support, the ECS obtains the service API information from the CAPIF core function via the Availability of service APIs event notification or Service Discover Response as specified in TS 23.222 [9].

5A-5B. The ECS contacts AAnF directly or via NEF to obtain the corresponding key KECS (KAF) of the UE(as defined in TS33.535).

6. The ECS verifies the MAC-I based on the KECS, when the verification is succeed, and if the UE is authorized to perform the operation. Then the ECS computes MAC-I over the response message using KECS, and sends a service provisioning response with the MAC-I to the UE.

### 6.11.3	Solution evaluation

  - This solution doesn’t apply to the case when there are multiple EECs in one UE.

This solution requires the new interface between ECS and AAnF.

****END OF CHANGES ***