3GPP TSG-SA3 Meeting #103-e 	S3-211489-r1

- e-meeting, 17th –  28th May 2021	Revision of xxxx

- Source:	Interdigital, Huawei, HiSilicon

- Title:	Conclusion for Key Issue #4

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is proposed to approve this pCR in TR 33.854.

# 2	References

# 3	Rationale

It is proposed to add a conclusion on Key Issue #4

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854.

***	BEGIN OF CHANGES	***

# 7	Conclusions

7.4	Conclusion on KI #4

For key issue #4 on Location information veracity and location tracking authorization:

- Solution #6, solution #8 and solution#12 are chosen as basis for normative work, based on the following key common principles:

- The UAS NF (aka UFES) receives location request from USS/UTM which may include a 3GPP UAV ID. If authorized, UAS NF provides USS/UTM with UAV location information including the 3GPP UAV ID (GPSI).

- To obtain UAV location information, the UAS NF uses location services (LCS) as supported by AMF/MME or GMLC. The Network-Assisted Positioning Procedure between the LMF and NG-RAN is selected for location information veracity.

- The UAS NF ensures that the USS/UTM is authorized to track the location of a given UAV before sending the UAV location information to USS/UTM. A USS/UTM is authorised to receive the location information of a group of UAVs in a particular geographic area or of an individual UAV if it has authorised the UAV(s) for service. Furthermore, a USS/UTM can be authorised to receive the data about all UAVs in a particular geographic area.

NOTE: The agreement for normative work is on key common principles and not on the other details of the solutions.

***	END OF CHANGES	***