3GPP TSG-SA3 Meeting #104-e 	S3-212538

# e-meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1152 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the description of KAUSF handling upon successful primary authentication-R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021.08.09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description of KAUSF handling in clause 6.1.1.1 and 6.2.2.1 is not correct. According to S3-191603 and the rest of this specification, the storage of KAUSF in the AUSF is mandatory after successful authentication in 5G AKA and EAP-AKA’. i.e. “shall be” rather than “may be” should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the description of KAUSF handling in clause 6.1.1.1 and 6.2.2.1 from “may be” to “shall be”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current description of KAUSF handling in clause 6.1.1.1 and 6.2.2.1 is not correct. It should be corrected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** 1st Change ****

6.1.1.1	General

The purpose of the primary authentication and key agreement procedures is to enable mutual authentication between the UE and the network and provide keying material that can be used between the UE and the serving network in subsequent security procedures. The keying material generated by the primary authentication and key agreement procedure results in an anchor key called the KSEAF provided by the AUSF of the home network to the SEAF of the serving network.

Keys for more than one security context can be derived from the KSEAF without the need of a new authentication run. A concrete example of this is that an authentication run over a 3GPP access network can also provide keys to establish security between the UE and a N3IWF used in untrusted non-3GPP access.

The anchor key KSEAF is derived from an intermediate key called the KAUSF. The KAUSF is established between the UE and HN resulting from the primary authentication procedure. The KAUSF shall be securely stored in the AUSF based on the home operator's policy on using such key. e.g. if the control plane solution for Steering of Roaming or UE Parameter Update procedures are supported by the HPLMN (see clauses 6.14 and 6.15)

NOTE A: For standalone non-public networks when an authentication method other than 5G AKA or EAP-AKA' is used, Annex I.2 applies.

NOTE 1:	This feature is an optimization that might be useful, for example, when a UE registers to different serving networks for 3GPP-defined access and untrusted non-3GPP access (this is possible according to TS 23.501 [2]). The details of this feature are operator-specific and not in scope of this document.

NOTE 2:	A subsequent authentication based on the KAUSF stored in the AUSF gives somewhat weaker guarantees than an authentication directly involving the ARPF and the USIM. It is rather comparable to fast re-authentication in EAP-AKA'.

NOTE 2a:	Void.

UE and serving network shall support EAP-AKA' and 5G AKA authentication methods.

NOTE 2b: It is the home operator's decision which authentication method is selected.

The USIM shall reside on a UICC. The UICC may be removable or non-removable.

NOTE 3:	For non-3GPP access networks USIM applies in case of terminal with 3GPP access capabilities.

If the terminal supports 3GPP access capabilities, the credentials used with EAP-AKA' and 5G AKA for non-3GPP access networks shall reside on the UICC.

NOTE 4:	EAP-AKA' and 5G AKA are the only authentication methods that are supported in UE and serving network, hence only they are described in sub-clause 6.1.3 of the present document. For a private network using the 5G system as specified in [7] an example of how additional authentication methods can be used with the EAP framework is given in the informative Annex B.

NOTE 5: For non-public network (NPN) security the Annex I of the present document provides details.

**** 2nd Change ****

6.2.2.1	Keys in network entities

Keys in the ARPF

The ARPF shall process the long-term key K and any other sensitive data only in its secure environment. The key K shall be 128 bits or 256 bits long.

During an authentication and key agreement procedure, the ARPF shall derive CK' and IK' from K in case EAP-AKA' is used and derive KAUSF from K in case 5G AKA is used. The ARPF shall forward the derived keys to the AUSF.

The ARPF holds the Home Network Private Key that is used by the SIDF to deconceal the SUCI and reconstruct the SUPI. The generation and storage of this key material is out of scope of the present document.

Keys in the AUSF

In case EAP-AKA' is used as authentication method, the AUSF shall derive a key KAUSF from CK' and IK' for EAP-AKA' as specified in clause 6.1.3.1. In case that 5G AKA is used as authentication method, the UDM/ARPF shall generate the KAUSF as specified in clause 6.1.3.2.The KAUSF shall be stored in the AUSF between two subsequent authentication and key agreement procedures.

When the AUSF stores the KAUSF, the AUSF shall store the latest KAUSF generated after successful completion of the latest primary authentication. The authentication is considered as successful and the AUSF shall store the latest KAUSF or replace the old KAUSF with the new KAUSF (if the SEAF(s) end up selecting the same AUSF instance for (re)authentication of the UE):

- in case 5G AKA is used as authentication method, when the RES* and the XRES* are equal (see clause 6.1.3.2.0, Step 11).

- in case EAP-AKA' is used as authentication method, when the AUSF sends an EAP-Success message to the SEAF (see clause 6.1.3.1, Step 10).

The AUSF shall generate the anchor key, also called KSEAF, from the authentication key material received from the ARPF during an authentication and key agreement procedure.

Keys in the SEAF

The SEAF receives the anchor key, KSEAF, from the AUSF upon a successful primary authentication procedure in each serving network.

The SEAF shall never transfer KSEAF to an entity outside the SEAF. Once KAMF is derived KSEAF shall be deleted.

The SEAF shall generate KAMF from KSEAF immediately following the authentication and key agreement procedure and hands it to the AMF.

NOTE 1: 	This implies that a new KAMF, along with a new KSEAF, is generated for each run of the authentication and key agreement procedure.

NOTE 2: 	The SEAF is co-located with the AMF.

Keys in the AMF

The AMF receives KAMF from the SEAF or from another AMF.

The AMF shall, based on policy, derive a key KAMF' from KAMF for transfer to another AMF in inter-AMF mobility. The receiving AMF shall use KAMF' as its key KAMF.

NOTE 3: The precise rules for key handling in inter-AMF mobility can be found in clause 6.9.3.

The AMF shall generate keys KNASint and KNASenc dedicated to protecting the NAS layer.

The AMF shall generate access network specific keys from KAMF. In particular,

-	the AMF shall generate KgNB and transfer it to the gNB.

-	the AMF shall generate NH and transfer it to the gNB, together with the corresponding NCC value. 
The AMF may also transfer an NH key, together with the corresponding NCC value, to another AMF, cf. clause 6.9.

-	the AMF shall generate KN3IWF and transfer it to the N3IWF when KAMF is received from SEAF, or when KAMF' is received from another AMF.

Keys in the NG-RAN

The NG-RAN (i.e., gNB or ng-eNB) receives KgNB and NH from the AMF. The ng-eNB uses KgNB as KeNB.

The NG-RAN (i.e., gNB or ng-eNB) shall generate all further access stratum (AS) keys from KgNB and /or NH.

Keys in the N3IWF

The N3IWF receives KN3IWF from the AMF.

The N3IWF shall use KN3IWF as the key MSK for IKEv2 between UE and N3IWF in the procedures for untrusted non-3GPP access, cf. clause 11.

Figure 6.2.2-1 shows the dependencies between the different keys, and how they are derived from the network nodes point of view.

Figure 6.2.2-1: Key distribution and key derivation scheme for 5G for network nodes

NOTE 4: The key derivation and distribution scheme for standalone non-public networks, when an authentication method other than 5G AKA or EAP-AKA' is used, is given in Annex I.2.3.

**** End of Changes ****