3GPP TSG-SA3 Meeting #105-e 	S3-214117-r1

- e-meeting, 08 - 19 November 2021											Revision of S3-20xxxx

- Source:	Lenovo, Motorola Mobility, Deutsche Telekom, Interdigital, Broadcom, Intel

- Title:	Conclusion to KI#1

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

This pCR provides a conclusion to Key Issue #1 in TR 33.864.

# 2	References

[1]	3GPP TR 33.864 Study on the security of Access and Mobility Management Function (AMF) re-allocation; Rel-17.

[2]	3GPP TS 23.502 Procedures for the 5G System (5GS); Rel-17.

[3]	S3-213035, ‘LS on full registration request in AMF reallocation via RAN’.

[4]	S2-2107860, ‘LS Response on full registration request message to be rerouted via RAN’.

# 3	Rationale

This pCR proposes a conclusion to Key Issue #1 taking the following main isolation cases into account.

- Isolation Case 1 (Does not share NAS security context between Initial AMF and Reallocated Target AMF):

  - Merger Possibility 1: Solution 1, 3, and 4 as stated in the conclusion of this document.

  - Merger Possibility 2: Merger of Solution 8 and Solution 6/7 can be taken as the baseline for the normative work, where solution 8 ensures availability of Requested NSSAI in advance and the Solution 6/7solves reroute via RAN issue. As there are few open ENs in solution 8, this merger possibility is yet to be confirmed and could not take precedence.

- Isolation Case 2 (Shares NAS security context in protected form between Initial AMF and Reallocated Target AMF):

Note: If sharing of NAS security context between initial AMF and reallocated AMF in a protected form is acceptable for the operators, then a merger possibility for Isolation Case 2 can be considered for the conclusion.

  - Merger Possibility 1: Solution 9, 10, 12 can be considered as baseline for the normative work with a common principle that the NAS security context can be protected and shared between initial and target AMF (with potential horizontal key derivation).

  - Using NSSF to provide security context for rerouting protection needs additional configurations in the NSSF, whereas if an AUSF is used, the reroute security can be based on a key available in the AUSF and it doesn’t need additional configurations. Therefore, which NF to be used as trusted NF for the reroute support can either be decided in the normative phase or during the meeting based on the preference from the operators.

Key issue#1 mainly discusses the issues with the isolation case specific to indirect AMF reallocation and reroute via RAN described in TS 23.502 Clause 4.2.2.2.3 Registration with AMF re-allocation. If any other isolation case needs to be introduced for this SID (e.g., no rerouting of registration request via RAN between initial and reallocated AMF), a suitable additional key issue is required for the new isolation case.

Clarification on Release specific impacts:

According to TS 23.502 Clause 4.2.2.2.3 Registration with AMF re-allocation, ‘If the security association has been established between the UE and initial AMF, to avoid a registration failure, the initial AMF shall forward the NAS message to the target AMF by executing step 7(A).’, so it is sufficient to have only Rel.17 specific adaptations and no adaptation is required for legacy UE or network to address key issue #1. Hence, the conclusion proposes normative recommendations specific to only Rel.17.

Further SA3 has sent an LS to SA2 [3] during 3GPP SA3#104-e meeting. Therefore, the conclusion proposes a related EN to take into account any additional clarifications as required for S3-213035 response.

Rerouting complete Registration Request:

SA2 LS [4] clarifies that, for Case 7(B) of clause 4.2.2.2.3 TS 23.502, the initial AMF is required to reroute via RAN, the complete registration request to the reallocated target AMF.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.864.

*****Start of Change*****

7	Conclusions

For Key Issue#1 the following has been recommended for the normative work:

- The security aspects for AMF reallocation and reroute via RAN can be supported by the 5G System from Rel. 17 onwards.

- The Rel.17 UE indicates in the Registration Request the support for AMF Reallocation and the indication from the network to UE to release an established NAS connection can be based on solutions 1 and 3.

- Based on solution 9,10 and 12, the 5GS shall enable the initial AMF to fetch the Reroute security context along with related information (identifier and token) from a core network function to protect the sensitive information (such as complete Registration Request) in the rerouted NAS message and to allow the reallocated AMF to authentictae itself to fetch the corresponding reroute security context. Further the reallocated AMF authenticates with the core network function and fetches the reroute security context to verify and the obtain the received protected reroute information. The NSSF in the core network faciliates provision of Reroute NAS security context.

- For the case, if the Reroute does not occur via RAN, solution 5 and 11 can be take as baseline and it requires a check with SA2 WG as the solutions involve architectural enhancements.

*****End of Change*****