3GPP TSG-SA3 Meeting #105-e  	S3-214218-r1

- e-meeting, 08 - 19 November 2021											Revision of S3-20xxxx

- Source:	Samsung, Interdigital, LG Electronics, Nokia, Nokia Shanghai Bell

- Title:	Proposal for U2NW relay authentication, authorization and key management

- Document for:	Approval

Agenda Item:	4.21

# 1	Decision/action requested

It is requested to approve this pCR for TS 33.503.

# 2	References

[1]	3GPP TR 33.847 Study on security aspects of enhancement for proximity-based services in the 5G System

# 3	Rationale

Based on outcome from SA3#104-e Ad-Hoc meeting, it was concluded that AUSF derives the PC5 anchor key (e.g., sol#1, sol#15, sol#30, sol#39) used for PC5 keys derivation and performing primary authentication during PC5 link establishment (e.g. Sol#1, Sol#10, Sol#30).

This pCR proposes a consolidated approach for L3 U2N Relay authentication, authorization and key management using the primary authentication for PC5 keys establishment. AUSF is used for key derivation and distribution of keys used for UE-to-network relay communication.

# 4	Detailed proposal

*****Start of 1st Change*****

# 2	References

[xx]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[yy]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services"

[zz]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[xy]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[aa]	3GPP TS 23.304: “Proximity based Services (ProSe) in the 5G System (5GS)”.

******2nd Change******

## 6.4	Security for 5G ProSe UE-to-Network Relay Communication

Editor’s Notes: This clause contains the description of the security for 5G ProSe UE-to-Network Relay Communication.

### 6.4.1	General

### 6.4.2	Security requirements

### 6.4.3	Security for 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay

#### 6.4.3.1	General

This sub-clause describes the security mechanisms for the L3 U2N Relay authentication, authorization and key management using the primary authentication for PC5 keys establishment. Network entities AMF, AUSF and UDM are involved for key derivation and distribution of keys used for UE-to-network relay communication.

#### 6.4.3.2	UE-to-Network Relay using ProSe Key Management Function

Editor’s Notes: This clause describes the security procedure that uses ProSe Key Management Function to authenticate/authorize UE during 5G ProSe UE-to-Network Relay Communication.

#### 6.4.3.3	Security procedure over Control Plane

The UE shall be provisioned with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [xx] clause 4.2.2. PCF shall provision the authorization policy and parameters for 5G UE-to-Network Relay Discovery and Communication as specified in 5.1.4 in TS 23.304 [aa]. Security materials related to the discovery shall be provisioned by 5GDDNMF.

Figure 6.4.3.3-1: UE-to-Network Relay security procedure based on primary authentication

0.	The Remote UE and relay UE shall be registered with the network.  The UE-to-Network relay shall be authenticated and authorized by the network to support as a relay UE. Remote UE shall be authenticated and authorized by the network to act as a Remote UE.

1.	The remote UE shall initiate discovery procedure using any of Model A or Model B method as specified in clause 6.3.1.2 or 6.3.1.3 of TS 23.304 [aa] respectively.

2-5.	After the discovery of the UE-to-Network relay, the Remote UE shall send a Direct Communication Request to the relay UE for establishing secure PC5 unicast link. The Remote UE shall include its security capabilities and security policy in the DCR message as specified in TS 33.536 [xx]. The message shall also include SUCI, Relay Service Code, Nonce_1. Upon receiving the DCR message, the Relay UE shall send the relay key request to the relay AMF, including the parameters received in the DCR message. The Relay AMF shall verify whether it is allowed to act as a relay UE. The relay AMF shall select AUSF based on SUCI and forward the key request to the AUSF in Nausf_UEAuthentication_Authenticate Request message.

6-7.	The AUSF shall retrieve the Authentication Vectors from the UDM and trigger primary authentication of the remote UE using existing procedure as specified in TS 33.501[xx]. This authentication is perfomed between the AUSF and the remote UE via the relay AMF and relay UE. AUSF shall not make the newly derived KAUSF as the latest KAUSF. At the remote UE, the newly derived KAUSF shall not be taken as latest KAUSF as NAS SMC procedure is not performed between remote UE and relay AMF.

8.	On successful primary authentication, the AUSF and Remote UE shall generate 5GPRUK and 5GPRUK ID as specified in Annex A.Y using the newly derived KAUSF.

9.	The AUSF shall generate the KNR_ProSe key as defined in Annex A.Z.

10-11. The AUSF shall send the 5GPRUK ID, KNR_ProSe, Nonce_2 in Nausf_UEAuthentication_Authenticate Response message to the UE-to-Network relay via relay AMF. Relay UE derives PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe, in a similar way as KNRP-sess is derived from KNRP, and confidentiality and integrity keys from KNRP-sess in TS 33.536[xy]. KNR_ProSe ID and Krelay-sess ID are established in the same way as KNRP ID and KNRP-sess ID in TS 33.536[xy].12. The UE-to-Network relay shall send the received 5GPRUK ID, Nonce_2 to the Remote UE in Direct Security mode command message

13-14.	The remote UE shall use the 5GPRUK ID to locate the 5GPRUK and shall generate the KNR_ProSe key to be used for Remote access via the Relay UE in the same way as defined in step 9. The Remote UE shall derive PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe the same way as defined in step 11. Remote UE shall send the Direct Security mode complete message to the UE-to-Network relay.

Further communication between Remote UE and Network takes place securely via the UE-to-Network relay.

##### 6.4.3.3.1	PC5 Key Hierarchy

Figure 6.4.3.3.1-1: PC5 Key Hierarchy for UE-to-Network Relay security

The different layers of keys (see Figure 6.4.3.3.1-1) are the following:

-	5GPRUK: The root credential derived from KAUSF that is the root of security of the PC5 unicast link.

-	KNR_ProSe: This is a 256-bit root key that is established between the two entities that communicating using NR PC5 unicast link. It may be refreshed by re-running the authentication to derive a fresh 5GPRUK.

-	Krelay-sess: This is the 256-bit key that is derived by UE from KNR_ProSe and is used derive keys that to protect the transfer of data between the UEs. The Krelay-sess is derived per unicast link same as KNRP-sess specified in TS 33.536 [xy]. During activated unicast communication session between the UEs, the Krelay-sess may be refreshed by running the rekeying procedure. The keys for confidentiality and integrity algorithms are derived directly from Krelay-sess. The 16-bit Krelay-sess ID identifies the Krelay-sess.

-	Krelay-int, Krelay-enc: The Krelay-int and Krelay-enc are used in the chosen confidentiality and integrity algorithms respectively for protecting PC5-S signalling, PC5 RRC signalling, and PC5 user plane data. These keys are equivalent to NRPIK and NRPEK as specified in TS 33.536 [xy]. They are derived from Krelay-sess and are refreshed automatically every time Krelay-sess is changed.

***** 3rd Change*****

Annex A (normative):
Key derivation functions

# A.X	KDF interface and input parameter construction

## A.X.1	General

All key derivations for 5G ProSe shall be performed using the key derivation function (KDF) specified in Annex B.2.2 of TS 33.220 [xx].

This clause specifies how to construct the input string, S, and the input key, KEY, for each distinct use of the KDF. Note that "KEY" is denoted "Key" in TS 33.220 [xx].

## A.X.2	FC value allocations

The FC number space used is controlled by TS 33.220 [xx], FC values allocated for the present document are in the range of 0xXX – 0xZZ.

# A.Y	5GPRUK derivation function

When deriving a 5GPRUK from KAUSF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXX;

-	P0 = SUPI;

-	L0 = length of SUPI.

-	P1 = relay service code;

-	L1 = length of relay service code.

The input key KEY is KAUSF.

SUPI shall behave the same value as parameter P0 in Annex A.7.0 of TS 33.501 [xx].

# A.Yy	5GPRUK ID derivation function

When deriving a 5GPRUK ID from KAUSF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xYY;

-	P0 = "PRUK-ID";

-	L0 = length of "PRUK-ID"; (i.e. 0x00 0x07)

-	P1 = SUPI;

-	L1 = length of SUPI.

The input key KEY is KAUSF.

SUPI has the same value as parameter P0 in Annex A.7.0 of TS 33.501 [xx].

# A.Z	KNR_ProSe derivation function

When deriving the KNR_ProSe from 5GPRUK key, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xZZ;

-	P0 = Nonce_2;

-	L0 = length of Nonce_2;

-	P1 = Nonce_1;

-	L1 = length of Nonce_1

The input key KEY shall be 5GPRUK key.

SUPI shall be have the same value as parameter P0 in Annex A.7.0 of TS 33.501 [xx].

******End of Change******