3GPP TSG-SA3 Meeting #103-e 	S3-214467

- e-meeting, 8 - 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0049 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Annex with Assets and Threats specific to SCAS SCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_SECOP |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is proposed to add a clause in an new Annex for the SCP network product class in TR 33.926 R17 for analysing the potential threats. When NFs and NF services communicate indirectly via the SCP, the SCP may receive tokens from consumer NFs (access token and client credentials assertion) or from the NRF (access token) which are to be forwarded to the producer NF for authorization of service access. If the SCP mistakenly forwards the tokens to a wrong pNF, or if the SCP mixes-up the received tokens and forwards the wrong tokens to the target pNF, there are the risks of authorization failure and the cNF’s tokens being exposed and exploited for misuse. Consequently, such risks will lead to the threats of Denial of Service, Information Disclosure, Elevation of Privilege, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added on Assets and Threats specific to SCP for analysing the potential threats related to the tokens forwarded by the SCP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of threat analysis for the purpose of the test case for security assurance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New Annex), X.2 (New), X.2.1 (New) X.2.1.1 (New), X.2.1.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR 0050 (S3-214469) introduces missing Annex X.1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** START OF CHANGES

Annex X (normative): 
Aspects specific to the network product class SCP

************** NEXT CHANGE

## X.2	Assets and Threats specific to SCP

## X.2.1	Threats related to tokens handled by the SCP

### X.2.1.1		Token forwarded to a wrong pNF instance

-	Threat name: Token forwarded to a wrong pNF instance

-	Threat Category: Denial of Service, Information Disclosure, Spoofing Identity, Elevation of Privilege

-	Threat Description:  According to TS 33.501 [14] clauses 13.4.1.3, the SCP is able to obtain the access token and/or client credentials assertion from the consumer NF, which are/is forwarded to the producer NF for authorization and/or authentication. If the SCP failed to select the correct target pNF instance due to e.g. mis-implementation or misconfiguration or weak internal interface protection, which results in the token being forwarded to the wrong pNF instance, there are the following threats:

-	The pNF instance receiving the access token will not be able to verify the access token, hence leading to authorization failure for indirect communication. Consequently, the access token, as a piece of sensitive information of the cNF, is exposed to a NF instance which is not meant to have it. This can result in denial of service, information disclosure, as well as waste of system resources. In worse case, if the NF instance receiving the access token wants to access the service granted in the token but not granted to the NF, it may impersonate the cNF to invoke the service of the target pNF using the received access token. This can result in spoofed identity and elevation of privilege.

-	The pNF instance receiving the client credentials assertion will not be able to verify the assertion, hence leading to failure of authenticating the cNF. Consequently, the credentials assertion, as a piece of sensitive information of the cNF, is exposed to a NF instance which is not meant to have it. This can result in denial of service, information disclosure, as well as waste of system resources. In worse case, if the NF instance receiving the assertion wants to access the service granted in the token but not granted to the NF, it may impersonate the cNF using the assertion for authentication with the target pNF, so as to facilitate the service access to the target pNF with the exposed access token. This can result in spoofed identity and elevation of privilege.

-	Threatened Asset: SCP Application, Access Tokens, Client Credentials Assertions, Service Messages forwarded/routed between NFs/NF services, Sufficient Processing Capacity

### X.2.1.2	Swapped token forwarded to the target pNF

-	Threat name: Swapped token forwarded to the target pNF

-	Threat Category: Denial of Service, Information Disclosure, Spoofing Identity, Elevation of Privilege

-	Threat Description:  When NFs and NF services communicate indirectly via the SCP, the SCP may receive multiple access tokens, which could come from multiple cNFs requesting services or from a single cNF requesting access to various services or network slices provided by the target pNFs. Particularly for indirect communication with delegated discovery, the SCP is delegated to request access tokens from the NRF for service requests from cNFs and then include the correct access token in each of the service requests from cNFs. If the SCP mixes-up the received multiple access tokens and includes the wrong access token in a service request, due to mis-implementation or misconfiguration, there are the following threats:

-	During indirect communication with delegated discovery, if the SCP mistakenly includes an access token of the requesting cNF destined for a pNF different from the target pNF (i.e. audience mismatch), the target pNF will fail to verify the access token, hence leading to authorization failure for indirect communication. Consequently, the access token, as a piece of sensitive information of the cNF, is exposed to a NF instance which is not meant to have it. This can result in denial of service as well as waste of system resources. In worse case, if the target NF wants to access the service granted in the token but not granted to the NF, it may impersonate the cNF to invoke the service of another pNF using the received access token. This can result in spoofed identity and elevation of privilege.

-	During indirect communication with delegated discovery, if the SCP mistakenly includes the access token of a cNF different from the requesting cNF (i.e. subject mismatch) or includes an access token of the requesting cNF for a service/slice different from the requested service/slice (i.e. scope mismatch), the target pNF will fail to verify the access token, hence leading to authorization failure for indirect communication. This can result in denial of service as well as waste of system resources.

-		During indirect communication with delegated discovery, if the SCP includes the correct access token but mistakenly includes a client credentials assertion of another cNF, the target pNF will fail to verify the assertion, hence leading to authentication failure for indirect communication. This can result in denial of service as well as waste of system resources.

-	Threatened Asset: SCP Application, Service Messages forwarded/routed between NFs/NF services, Sufficient Processing Capacity

************** END OF CHANGES