- 3GPP TSG-SA3 Meeting #107Adhoc -e	S3-221434-r2

- e-meeting, 27th June – 1st July, 2022

- Source:	ZTE

- Title:	New solution about the roaming AKMA architecture of the AF in the VPLMN

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

This contribution proposes proposes a new solution about the roaming AKMA architecture of the AF in the VPLMN.

# 2	References

[1] 	3GPP TR33.737 V0.1.0

# 3	Rationale

This new solution proposes the roaming AKMA architecture of the AF in the VPLMN.

# 4	Detailed proposal

********************Start of 1st Change******************

## 6.Y	Solution #Y: Roaming AKMA architecture of the AF in the VPLMN

### 6.Y.1	Introduction

This solution addresses KI#1. This solution is proposed for the cases of regulatory control point deployment in which the VPLMN supports AKMA when AF located in the VPLMN.

6.Y.2	Solution details

Figure 6.Y.1-1 shows the SBA architecture that UE is roaming in a VPLMN and accesses the application deployed in the VPLMN. The home network (HPLMN) needs an agreement with the service network (VPLMN) to use the AKMA service deployed in the home network. In this scenario, the service network can sense the AKMA service used by the UE. For this scenario, AF is located inside the VPLMN and needs to find the AAnF of the home network through the NEF of the home network.

Figure 6.Y.1-1: The SBA architecture that UE is roaming in a VPLMN and accesses the application deployed in the VPLMN

Figure 6.Y.1-2 shows the architecture that UE is roaming in a VPLMN and accesses the application deployed in the VPLMN. As described in clause 6.3 of the 3GPP TS 33.535[2], the AF needs to obtain the KAF from the AAnF through the NEF. Therefore, the regulatory control point in figure  6.Y.1-4 can be AF in the VPLMN.

Figure 6.Y.1-2: The architecture that UE is roaming in a VPLMN and accesses the application deployed in the VPLMN(*regulatory control point)

Editor’s Note: The details of the AF being a regulatory control point for LI are FFS.

Editor’s Note: How LI requirements are to be fulfilled is FFS.

6.Y.3	Solution Evaluation

TBD

********************End of 1st Change******************