3GPP TSG-SA3 Meeting #107e AdHoc 	draft_S3-221540-r1

- e-meeting, 27 June - 1 July 2022							   Revision of S3-221540+221398+221455

- Source:	Xiaomi, Huawei, HiSilicon, ChinaTelecom

- Title:	33.893: New Key Issue on Privacy

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed new key issue to be added in TR 33.893.

# 2	References

[1]	SP-220534	New SID on Security Aspects of Ranging Based Services and Sidelink Positioning

[2]	S3-221537	33.893: Draft Skeleton

# 3	Rationale

The SID on Security Aspects of Ranging Based Services and Sidelink Positioning [1] was approved at SA#96e. The skeleton of the Technical Report for the study (TR 33.893) is proposed in S3-221537 [2]. This pCR intends to propose a new key issue on privacy protection for Ranging/Sidelink Positioning services based on the skeleton of TR 33.893.

# 4	Detailed proposal

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]					3GPP TS 22.261: "Service requirements for the 5G system".

[y]	3GPP TR 23.700-86: "Study on Architecture Enhancement to support Ranging based services and sidelink positioning".

*************** Start of the 2nd Change ****************

# 5	Key issues

5.X	Key issue #X: Privacy protection for Ranging/SL Positioning services

### 5.X.1	Key issue details

As the information of almost all Ranging/Sidelink Positioning services is related to location, all the UEs participating in Ranging/Sidelink Positioning, including the reference UE, target UE, assistant UE, etc., may need to disclose its location information to others. If such privacy sensitive information is not well protected, the UE’s privacy could be compromised. Among the requirements defined for Ranging services in clause 6.37.2 of TS 22.261 [x], there are following requirements concerning privacy protection for Ranging services:

The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during ranging.

The 5G system shall be able to ensure that user privacy is not violated during ranging, e.g., subject to regional or national regulatory requirements.

Privacy protection is also raised in clause 4.1 of TR 23.700-86 [y] as one of the architecture assumptions for Ranging/SL Positioning services and is tasked for SA3 to study. In multiple solutions (e.g. solutions #6, #9, #13, #18, #21, #23, #24, #25) of TR 23.700-86 [y], privacy is considered as an issue to be addressed, either during discovery, or during Ranging/SL positioning procedure, or for service exposure.

### 5.X.2	Security threats

When UE’s identifiable information (e.g. SUPI, UE’s application layer ID) is disclosed to undesired/malicious UEs during discovery or during communication for Ranging/SL positioning, the UE’s behaviour will become trackable to others. Hence the UE’s privacy could be violated.

When the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both, or assistant data) and/or the associated UE’s identity are disclosed to undesired/malicious UEs or undesired network functions during communication for Ranging/SL positioning, the UE’s whereabouts and/or movements will become traceable to others. Hence the UE’s privacy could be violated.

When the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both) and/or the associated UE’s identity are disclosed to undesired entities (e.g. a third party server or third party UE) during service exposure for Ranging/SL positioning, the UE’s whereabouts and/or movements will become traceable to others. Hence the UE’s privacy could be violated.

If the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both) is obtained by another UE or network function or third party server without being granted user consent, the UE’s privacy may be violated subject to regional regulations.

### 5.X.3	Potential security requirements

The 5G Ranging/SL Positioning system should be able to ensure that the UE’s identifiable information (e.g. SUPI, UE’s application layer ID) is protected from being disclosed to undesired entities during discovery for Ranging/SL positioning.

The 5G Ranging/SL Positioning system should be able to ensure that the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both, or assistant data) and/or the associated UE’s identity are protected from being disclosed to undesired entities during communication for Ranging/SL positioning.

The 5G Ranging/SL Positioning system should be able to ensure that the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both) and/or the associated UE’s identity are protected from being disclosed to undesired entities during service exposure for Ranging/SL positioning.

The 5G Ranging/SL Positioning system should be able to ensure that the entity requesting the UE’s Ranging/SL positioning information (e.g. distance measurement, direction measurement, or both) is granted user consent before obtaining the requested information.

*************** End of the Changes ****************