3GPP TSG-SA3 Meeting #107-e 	draft_S3-220933-r2

- e-meeting, 16 - 20 May 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Requirement to KI on roaming hub

- Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Requirement proposal.

# 2	References

[1]	3GPP TR 33.875

# 3	Rationale

Requirement proposal.

# 4	Detailed proposal

******************** START OF CHANGES

### 5.10.3	Security requirements

Existing requirements in TS 33.501 on the SEPP shall continue to apply unless there are explicit exceptions. Specifically, the requirements on edge protection of the PLMN shall still apply. Existing NFs and SCP shall be impacted as least as possible.

- -	The external entity shall use its own unique credentials to authenticate itself towards the remote SEPP.

-	Any connection between NFs or SCPs of a PLMN and a SEPP shall be confidentiality, integrity and replay protected.

- 	It shall be possible for the remote SEPP to verify that the external entity has been authorized by the PLMN to operate the SEPP.

-  	The solution shall enable the external entity to operate SEPPs for multiple PLMNs in a way that isolates the SEPP instances operated for different PLMNs from each other for security reasons. Specifically, if one of the instances gets corrupted or otherwise malfunctions, other instances shall remain unaffected as much as possible.

******************** END OF CHANGES