3GPP TSG-SA3 Meeting #107-e 	S3-221126-r1

- e-meeting, 16 - 20 May 2022

- Source:	Samsung

- Title:	New solution on UDM initiated Re-authentication based on AUSF request

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new solution on the need for home network triggered re-authentication.

# 2	References

[1]	3GPP TS 33.xxx Security aspects of home network triggered re-authentication.

# 3	Rationale

This contribution proposes a solution to the corresponding KI on home network triggered re-authentication in S3-221125

# 4	Detailed proposal

*****Start of 1st Change*****

# References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 33.501:	 “Security architecture and procedures for 5G system”.

*****End of 1st Change*****

*****Start of 2ndChange*****

## 6.Y	Solution #Y: UDM initiated primary authentication based on AUSF request

### 6.Y.1	Introduction

This solution addresses KI#X. The UDM triggers re-authentication of the UE, if an internal NF request (AUSF) to initiate re-authentication to refresh the UE specific home key (KAUSF).

### 6.Y.2	Solution details

A new primary authentication may require for certain events at the network, resulting in refresh of the latest home key KAUSF. In such scenarios, an internal Network Function (AUSF) requests the UDM to trigger the re-authentication procedure.

Figure 6.Y.2-1: UDM initiated primary authentication

- The primary authentication is performed as specified in TS 33.501[xx]. After successful authentication, KAUSF is derived at the AUSF and at the UE.

- AUSF determines (due to long time availability of same key, etc) the need of refreshing the KAUSF key.

- If the AUSF determines that there is a need to refresh the KAUSF, it decides to request UDM to initiate the primary authentication to refresh the home key KAUSF. AUSF sends a re-authentication request to the UDM by providing SUPI of the UE.   Prior to sending the request to UDM, the AUSF checks whether the primary authentication for the UE to be initiated or request to be rejected, based on the operator policy. Operator policy includes the details of the wait period for the new request, after the last successful authentication.

- Upon receiving the re-authentication request from the AUSF, UDM checks (if the check is not performed at the AUSF) whether the primary re-authentication for the UE to be initiated or request to be rejected, based on the operator policy. Operator policy includes the details of the wait period for the new request, after the last successful authentication.

- If the operator policy allows, then the UDM requests the AMF currently serving the UE to initiate the primary authentication for the UE. On receiving the request, the AMF initiates the primary authentication.

- Upon receiving the request from the UDM, the AMF(SEAF) initiates the primary authentication as described in clause 6.1.2 of TS 33.501 [xx], resulting in generation of fresh key material in the UE and in the network as described in clause 6.2 of TS 33.501 [xx], if the primary authentication is performed successfully.

### 6.Y.3	Solution Evaluation

TBD

*****End of Change*****