3GPP TSG-SA3 Meeting #107-e 	S3-220730

# e-meeting, 16 - 21 May 2022																revision of S3-220390

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Resolution of EN on authorization method negotiation per KI7-Sol9

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 33.875

# 3	Rationale

Proposal which would also allow the deletion of the EN.

# 4	Detailed proposal

**************** START OF CHANGES

### 6.9.3	Evaluation

This solution addresses the threats and requirements of Key issue #7: "Authorization mechanism negotiation".

This solution addresses the use case that the vPLMN has only implemented static authorization. The NF Service Producer needs to support whichever authorization mechanism is determined by the hNRF, i.e. using different authorization mechanisms depending on the roaming partner.

The vNRF in the vPLMN sends the vPLMN authorization Capability to the hNRF in the hPLMN during the discovery procedure in roaming scenario. The hNRF selects the Final authorization mechanism supported by both the received vPLMN authorization Capability and the hPLMN authorization Capability, and sends the Final authorization mechanism to the NFc. Thus, the authorization mechanism among both roaming parties is consistent and a roaming interaction failure is avoided.

The assumed use case in this solution is not in line with specification requirements. Since Rel-15, PLMNs shall support OAuth2.0 for authorization. Thus, even if vPLMN has only implemented static authorization, it still can indicate in the discovery phase whether OAuth2.0 is required or not.

**************** NEXT CHANGES

## 7.7	KI#7: Authorization mechanism determination

### 7.7.1	Analysis

The assumption in the key issue is not in line with specification requirements. Since Rel-15, PLMNs shall support OAuth2.0 for authorization. Thus, if vPLMN has only implemented static authorization, it still can indicate in the discovery phase that OAuth2.0 is not required. Then it depends on the hPLMN to accept static authorization as the alternative authorization method for accessing its services.

### 7.7.2	Conclusion

The problem can be addressed with the existing methods. No additional solution is needed.

**************** END OF CHANGES