3GPP TSG-SA3 Meeting #107-e 	S3-220796

# e-meeting, 16 - 20 May 2022											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	New solution for part 1 of KI#2

# Document for:	Approval

Agenda Item:		5.5 FS_eNS2_SEC

# 1	Decision/action requested

Include the solution in TR33.874

# 2	References

[1]

# 3	Rationale

This contribution proposes a new solutoin to address the KI on DoS to NSAC procedure.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (All text are new) ***

## 6.Y	Solution #Y: NSAC based on Usage of Network Slices

### 5.1.1	Introduction

This solution is to perform NSAC based on the real usage status of a network slice. In other words, if a network slice is not being used by a UE, the UE will not be counted at NSACF.

### 5.1.2	Solution details

0. UE has registered to a PLMN and assuming the UE is allowed to access slices S-NSSAI-1 and S-NSSAI-2. UE is counted as being registed at NSACF for both S-NSSAI-1 and S-NSSAI-2, according to the procedure in TS23.502.

1. In case a network slice (e.g. S-NSSAI-2) is not being used (e.g. no PDU session in the S-NSSAI-2), the serving AMF initiates the Number of UEs Availability Check and Update procedure as specified in the clause 4.2.11.2 of TS23.502, where NSACF will update its quota usage status (-1 in S-NSSAI-2) and responses to the AMF.

2. AMF stores the UE status at the network slice (i.e. the UE is no longer be counted for S-NSSAI-2’s quota).

3. AMF may update the UE with the UCU procedure that S-NSSAI-2 has been removed from the allowed S-NSSAI.

### 5.1.3	Evaluation

TBC

***	END OF CHANGES	***