- 3GPP TSG-SA3 Meeting #108Adhoc-e	draft S3-222636-r1

- e-meeting, 10th – 14th October, 2022

- Source:	ZTE

- Title:	Address EN and add evaluation for solution 3

- Document for:	Approval

- Agenda Item:	5.6

# 1	Decision/action requested

This contribution proposes to address ENs and update evaluation in solution 3 in TR 33.737.

# 2	References

[1] 	TR33.737 v0.2.0

# 3	Rationale

There is two editor’s notes in the solution 3:

“Editor’s Note: The details of the UPF being a regulatory control point for LI are FFS.”

“Editor’s Note: How LI requirements are to be fulfilled is FFS.”

The UPF in the VPLMN can obtain related decryption keys from the AF in the HPLMN base on the laws and regulations of the HPLMN when the data between the UE and application had been encrypted. So, the requirement of the KI#1(case 1 and case 3) can be met. It is proposed to add the evaluation part for the solution.

# 4	Detailed proposal

********************Start of 1st Change******************

## 6.3	Solution #3: Roaming AKMA architecture of the AF in the HPLMN

### 6.3.1	Introduction

This solution addresses KI#1( case 1). This solution is proposed for the cases of regulatory control point deployment when AF located in the HPLMN.

### 6.3.2	Solution details

Figure 6.3.1-1 shows the SBA architecture that UE is roaming in a VPLMN and accesses the application deployed in the HPLMN. When UE is roaming in a VPLMN and accesses an application deployed in the HPLMN, the VPLMN does not need to detect whether the UE uses the AKMA service.

Figure 6.3.2-1: The SBA architecture that UE is roaming in a VPLMN and accesses the application deployed in the HPLMN

Figure 6.3.1-2 shows the architecture that UE is roaming in a VPLMN and accesses the application deployed in the HPLMN. The UE accesses the home network deployment application. Like the third party deployment service,  if the data between the UE and application had not been encrypted, the VPLMN can detect the unencrypted data flow through UPF in the VPLMN. If the data between the UE and application had been encrypted, the UPF in the VPLMN first identify that data packets are AKMA services, and then obtain related decryption keys from the AF in the HPLMN base on the laws and regulations of the HPLMN. In this case, the function that enables LI should be UPF in the VPLMN.

Editor’s Note: The details of the UPF identifying the data packets to AKMA services are FFS.

Figure 6.3.2-2: The architecture that UE is roaming in a VPLMN and accesses the application deployed in the HPLMN (*function that enables LI)

6.3.3	Solution Evaluation

This solution addresses key issue#1 (case 1) and fulfils the requirements in case that the AF is located in the HPLMN.

The solution proposes to obtain related decryption keys from the AF in the HPLMN base on the laws and regulations of the HPLMN when the data between the UE and application had been encrypted.

********************End of 1st Change******************