3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222722, 924? merged in draft_S3-222681-r3

- e-meeting, 10 - 14 October 2022

- Source:	US National Security Agency, NIST, CISA ECD, Lenovo, CableLabs, InterDigital, AT&T, Johns Hopkins University APL, CIS

- Title:	ZT Tenet 3

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

SA3 is asked to approve this addition to TR 33.894

# 2	References

[1]	3GPP TR 33.894, ‘Study on applicability of the Zero Trust Security principles in mobile networks

(Release 18)’.

[2]	NIST Special Publication 800-207, ‘Zero Trust Architecture’, August 2020.

# 3	Rationale

This submission contains a gap analysis comparing the NIST zero trust tenets to current 3GPP security mechanisms for the 5G Core. For this analysis, some thought was put into how the tenets specifically apply to a 5G Core since the NIST Special Publication 800-207 Zero Trust Architecture utilizes a user-to-enterprise example for applying the zero trust tenets. Further it is worth noting that, according to [2], ‘The tenets attempt to be technology agnostic.’.

# 4	Detailed proposal

***	BEGINNING OF 1st CHANGE   ***

5.Y	Tenet #3: Access granularity

5.Y.1	Description

According to tenet 3 of [2], a zero trust architecture has to adhere to the principle that "Access to individual enterprise resources is granted on a per-session basis". This tenet is about access authorization to resources.

5.Y.2	Relevant security mechanisms

All the security mechanisms specified in TS 33.501 [a] related to SBA security. In addition, all the security mechanisms for UE's access 5GC in particular the security and authorization mechanism for PDU session establishment procedure.

5.Y.3	Evaluation

Authorization is one of the most important mechanism for protecting enterprise resources. From this aspective, 5GC indeed provides the necessary authorization and authentication mechanisms for NF to be deployed in enterprise network.

In the 5GC, one can assimilate the notion of session from [2] to the TLS session, considering that TLS is used for the SBI interface protection as defined in TS 33.501 [a]. In fact, NF consumers and producers are first required to mutually authenticate during the TLS session establishment via certificates. Then the NF consumer may be required to present an OAuth2.0 authorization token to the producer in the service request within the TLS tunnel. These two mechanisms provide the necessary tools for an operator to control authorization at almost a service invocation level. In facts, via the claims in the OAuth tokens, an operator can restrict access to single instances, to particular services, to particular type of producers, to indicate the restrictions for different resoures in 5GC NFs, etc. In addition, via the optional "additional scope", the operator can further restrict the allowed resources and the allowed actions. This means that besides the per-TLS session authorization, 5GC core network provides much concrete granularity on the authorization based on the OAuth 2.0 mechanism.

For UE access, one can assimilate the notion of session from [2] to the PDU session. From this perspective, the PDU session exchange procedure takes place over a secure NAS channel established following a successful mutual authentication procedure with the UE. The session is can be authorized based on operator policies related to subscription, data network, etc. and provisioned in the UDM or PCF. Furthermore, authorization can be further enhanced through a run of the secondary authentication procedure with the external Data Networks towards which the session is being established as specified in clause 11 of TS 33.501 [a].

However, the usage of such security mechanisms is left to the discretion of the operator and will depend on the deployment context. Nevertheless, based on the current standards, the operator of a 5G Core has the means to impose very granular and tight restrictions for access to resources. Therefore, no further actions from standard perspective are identified for this tenet.

***	END OF 1st CHANGE   ***