3GPP TSG-SA3 Meeting #108Adhoc-e 	S3-222704r1

- e-meeting, 10th – 14th October 2022                                                               Revision of S3-22xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	solution 1 updates

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is requested to approve the updates on the solution1.

# 2	References

[X] 	3GPP TR 33.741 "Study on home network triggered primary authentication (HONTRA)".

- 3	Rationale

Observation 1: Regarding the EN " Editor's Note: Either to use reauthentication required cause or to use a new service is FFS.", we believe that the existing AMF services "Namf_Communication" can be reused and there is no need to define a new service at the AMF. The AMF's Namf_Communication service always takes care of all the authentication-related aspects, which includes reauthentication itself.

Therefore, the same service can be reused for the UDM-initiated reauthentication. Therefore, we propose to remove this EN.

Observation 2: Diagram and text contains callback URI details however, it should be defined by CT.

Therefore, removing the callback URI details from step 1.

Observation 3: Section 5.1.2.3 EN resolutions:

Editor’s Note: Which NFc invokes the UDM service is FFS.

>> AAnF can invokes the UDM service .

Editor's Note: Either use the existing subscribe API at the UDM or define a new request is FFS

>> Following note is added " NOTE: stage3 will decide either to use existing API or define a new API at UDM.

Editor's Note: The factors considered by the AAnF to determine if a UE re-authentication is required or not is FFS.

Editor's Note: How the AAnF knows such factors which helps to determine re-authentication requirement is FFS.

>> Whenever UE is reauthenticated, the AUSF informs AAnF and AAnF updates the context. Therefore, AAnF is well aware of the UE authentication timestamp. Secondly, always a single AAnF is assigned for a UE.

Whenever Kaf has expired at the (internal or external)AF and the AF wants to refresh the Kaf via 5GC, the AF asks AAnF for UE reauthentication so that Kaf can be refreshed. The AAnF maintains the "wait time before initiating new reauthentication". For example, suppose a configured value of " wait time before initiating new reauthentication" is 30 min then

- AAnF shall not invoke the UDM reauthentication service/API if primary (re) authentication is performed less than 30 min and returns an error to AF.

- If the interval is 30 minutes or more, then AAnF invokes the UDM service/api to start the primary reauthentication.

- 4	Detailed proposal

**** START OF CHANGE ****

- 5.1	Solution #1: HN triggering primary authentication for various scenarios

- 5.1.1	Introduction

This solution provides a framework where the home network can trigger a primary authentication due to various scenarios, i.e.,

-	SOR/UPU Counter wraparound;

-	EPC to 5G interworking where KAUSF would not be available at UE and AUSF. And due to this, a few services will not work;

-	Any other scenarios where any authorized NF can invoke the primary reauthorization.

The solution provides an advanced detection solution where the SoR/UPU wraparound situation is detected in advance, i.e., the AUSF detects that the UPU/SoR counter will reach its max value in the next SoR/UPU case and takes precautionary measures in advance.

- 5.1.2	Solution details

- 5.1.2.1	Procedure for detection of SoR/UPU Counter wraparound in advance and perform reauthentication

Figure 6.X.2.1: detection of SoR/UPU Counter wraparound in advance and perform reauthentication

- The UE is authenticated and registered at the 5GC.

2. The UDM decides to perform the UE parameter update procedure or SoR procedure. Therefore, the UDM invokes the Nausf_UPUProtection or Nausf_SoRProtection procedure. The AUSF tries to increment the Counterupu/CounterSoR and figures out that the Counterupu/CounterSoR is about to wrap around or will reach the max value at the next trigger. Therefore, AUSF shall provide an indication of "Counterupu Reaching Max value" or "/CounterSoR Reaching Max value" to UDM along with the successful result of the Nausf_UPUProtection or Nausf_SoRProtection procedure. This alerts the UDM that the current UPU/SoR procedure is OK to continue, but the next follow-up UPU procedure update will cause a wraparound failure. The UDM shall store the received indication.

3. The UDM completes the UPU/SoR procedure as defined in TS 33.501[3].

4. If "Counterupu Reaching Max value" or "CounterSoR Reaching Max value" indication is received in step 2, the UDM sends a notification to the AMF with SUPI and the "reauthentication required" flag set to true.

Note: In the existing specs, UDM contacts AMF on Namf_communication service to trigger the re-registration required or reauthentication required with deregistration first. Therefore, UDM can use the same Namf_communication service for reauthentication without performing the deregistration.

5. Based on the received indication, the AMF shall start the primary reauthentication procedure.

6. AMF shall invoke Nausf_UEAuthetication_Authenticate Request with SUPI and SN-Name.

7. The AUSF sends the Nudm_UEAuthentication_Get Request with SUPI and SN-Name to the UDM.

Steps 8-10 are the same as defined in TS 33.501[3] clause 6.1.3. After Key KAUSF is generated, the CounterUPU and CounterSoR shall be reset. Therefore, any further UPU/SoR trigger at the UDM will be successful.

- 5.1.2.2	Reauthentication due to EPC to 5G mobility

When UE moves from EPC to 5GC, the AMF performs Registration with UDM and invokes Nudm_UECM_Registration API. UDM should check if there is no authentication result stored in the UDM, then the UDM shall send a notification to the AMF with SUPI and the "reauthentication required" flag set to true as defined in clause 5.1.2.1. Step 4.  It ensures KAUSF is available in AUSF and UE when a user moves from EPC to 5GC so that different services work smoothly, i.e., SoR, UPU, and/or a user accessing the AKMA AF.

- 5.1.2.3	Reauthentication invoked by other AAnF

If the AAnF wants to invoke the UE Reauthentication, the NF shall request the UDM to perform re-authentication.

Whenever KAF has expired at the (internal or external) AF and the AF wants to refresh the KAF via 5GC, the AF request AAnF for UE reauthentication so that KAF can be refreshed. The AAnF maintains the "wait time before initiating new reauthentication". For example, suppose a configured value of "wait time before initiating new reauthentication" is 30 min then AAnF shall not invoke the UDM reauthentication service/API if primary (re) authentication is performed less than 30 min ago and returns an error to AF. If the interval is 30 minutes or more, then AAnF invokes the UDM service/api to start the primary reauthentication. Alternatively, "wait time before initiating new reauthentication" can be maintained at the UDM as well.

Figure 6.X.2.3: Reauthentication invoked by other NFs

- The AAnF wants to perform re-authentication. Therefore, NF shall send a Nudm_EventExposure_Subscribe request to UDM with a new flag for Reauthentication Required.

2. The UDM authorizes the request based on existing means (e.g. Oauth2.0).

3. If authorization is successful, the UDM checks whether the primary authentication for the UE to be initiated or request to be rejected, based on the operator policy. Operator policy includes the details of the "waiting period before initiating new reauthentication". If the check is passed, the UDM shall invoke the reauthentication procedure as defined in Steps 4 to 9 of clause 5.1.2.1.

- 5.1.3	Solution Evaluation

TBD.

**** END OF CHANGE ****