# 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222470

# e-meeting, 10th – 14th October, 2022

# Source: 	Huawei, HiSilicon

# Title:	Security threat and requirement in MOCN network sharing scenario

# Document for:	Approval

Agenda Item:	5.23

1	Decision/action requested

It is proposed to approve the change described in this document.

2	References

[1]	3GPP TR 33.883: " Study on security enhancements for 5G multicast-broadcast services phase 2".

3	Rationale

In R17, the security protection for broadcast is provided in service layer. In R18, optimization is studied for the MOCN network sharing scenario. Similarly, the 5G system needs to protect the traffic in service layer in MOCN network sharing scenario.

4	Detailed proposal

*** 1st CHANGE ***

5.1.1	Key issue details

In MOCN network sharing scenario, multiple CNs are connected to the same NG-RAN. As documented in TR 23.700-47 [2], the efficiency of resource utilization for the same broadcast content is studied. For the same broadcast content, the AF will set up multiple broadcast MBS sessions towards those CNs. Each CN will deliver the same content towards the same shared NG-RAN node. The NG-RAN node only delivers one copy of the broadcast content over the air.

As specified in clause W.4 of TS 33.501 [3], user-plane procedure is applicable for broadcast service. MBSTF may protect the traffic transmission with encryption and/or integrity. The security protection of MBS traffic is supported in service layer. In MOCN network sharing scenario, the multiple CNs may enable their own security towards the content. The UE will receive the MBS keys from their PLMN. However, the NR-RAN broadcasts only one copy of the content. The security impact needs analysis if security are activated for the same content to be provided to 5G MOCN network sharing scenarios. For example, UEs from PLMN1 may be unable to decipher the content if the NG-RAN node chooses to broadcast the ciphered content from the CN of PLMN2.

If the content is protected using different CN-specific keys, then UEs not having the key will fail to properly process the content, should the network send only one of the copies.

5.1.2	Security threats

Reusing the existing security procedure in service layer may cause processing failure in UEs in MOCN network sharing scenario. However, if the content is broadcasted without protection, the attackers may fool the user of the content from the actual source by modification and replay of messages, e.g. replace the actual content with a fake one.

5.1.3	Potential security requirements

The 5G system shall provide the means to protect the traffic in service layer in MOCN network sharing scenario.

*** END OF 1st CHANGE***