3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222482

# e-meeting, 10th - 14th October 2022

# Source:	Huawei, HiSilicon

# Title:	New Key Issue on Security and privacy of multi-path transmission for UE-to-Network Relay

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

Approve the new KI proposal to TR 33.740

2	References

[1]

3	Rationale

The contribution proposes to add a Key Issue about Security and privacy of multi-path transmission for UE-to-Network Relay. Based on the background that SA2 has been established some conclusion regarding multi-path issue and the security aspects should coordination between SA2 and SA3, as specified in KI#5 of TR 23.700-33. For the above reasons, it is necessary to study whether security enhancement is needed.

4	Detailed proposal

***  BEGINNING OF 1st CHANGES  ***

5.X	Key Issue #X: Security and privacy of multi-path transmission for UE-to-Network Relay

5.X.1	Key issue details

Key Issue #5 in TR 23.700-33 [2] includes the architecture study about multi-path transmission for UE-to-Network Relay, focusing on the authorisation, rule provisioning and procedures of the multi-path scenario with one direct network path and one U2NW connection. This Key Issue is intended to cover the related security and privacy aspects of the multi-path scenario (one direct Uu communication and one indirect U2NW relay path).

5.X.2	Security threats

If security policies are not aligned on the direct Uu and L3 UE-to-Network relay multi-path transmission, the attacker could intercept the message exchange during the path switch procedures over the indirect link without confidentiality and/or integrity protection.

If security policies are not aligned on the direct Uu and L3 UE-to-Network relay multi-path transmission, the attacker could intercept the data transferred over the indirect link without confidentiality and/or integrity protection.

If security and/or privacy measures are not considered and applied during and/or after the multi-path transmission establishment procedures, an attacker can track the remote UE by linking the direct and indirect communications.

5.X.3	Potential security requirements

The 5G system shall support the means to consistently enforce security over direct Uu and L3 UE-to-Network Relay multi-path transmission.

The 5G system shall protect the privacy of the Remote UE during and/or after the multi-path establishment between direct Uu and indirect UE-to-Network Relaying paths.

***	END OF 1st CHANGES	***