3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222796

# e-meeting, 10 - 14 October 2022

# Source:	Ericsson

# Title:	Resolve some ENs for Solution4

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This document proposes to resolve some ENs for solution4.

# 2	References

[1]	3GPP TR 23.700-33

[2]	3GPP TR 33.740

# 3	Rationale

This contribution proposes to resolve the following ENs in Solution #4: PC5 security establishment when L3 UE-to-UE relay is out of coverage.

- Editor’s Note: Further provisioned parameters e.g., PC5 security policies of UE-to-UE relay, public keys are FFS.

For 5G ProSe UE-to-Network Relay scenario, PC5 security policies can be provisioned by PCF during service authorization procedure, or by 5GDDNMF during discovery request procedure, or by 5G PKMF in case 5G Layer-3 U2N security is established over user plane.

It is our view that similar provisioning procedure of PC5 security policies can be applied for UE-to-UE Relay case.

It is proposed that PC5 security policies can be provisioned to the UEs, while the 5GC NFs that support the provisioning can be decided during Normative phase. Whether or not public keys are provisioned by the 5GC NFs would be covered by other ENs.

- Editor’s Note: Further and remaining messages needs to be updated and clarified in step 8.

Direct Communication Accept message is required to complete the PC5 connection establishment procedure.

It is proposed to add Direct Communication Accept message in the flow.

- Editor’s Note: It is FFS whether PC5 security set up procedure between target UE and Relay UE is performed after or in parallel to the PC5 security set up procedure between source UE and Relay UE.

SA2 concluded that for Layer-3 UE-to-UE Relay:

- 	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

I.e.  PC5 link setup between target UE and Relay UE is not parallel to the PC5 link setup between source UE and Relay UE.

It is proposed to remove this EN.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.4	Solution #4: PC5 security establishment when L3 UE-to-UE relay is out of coverage

### 6.4.1	Introduction

This solution addresses Key issue #2: Security of UE-to-UE Relay and Key issue #3: Authorization in the UE-to-UE Relay Scenario. This solution addresses a L3 UE-to-UE relay.

For UE-to-UE relay use cases, the L3 UE-to-UE relay may be in or out of 3GPP coverage. This solution provides a mechanism for PC5 security setup procedure between a source UE or target UE and a L3 UE-to-UE relay when the L3 UE-to-UE relay is out of 3GPP coverage.

This solution assumes long term credentials are provisioned into the UE(s) and form the root of the security of the PC5 unicast link as specified in TS 33.536 [9].

This solution proposes to use authorization tokens as in OAuth 2.0 to indicate that a source UE or a target UE or a L3 UE-to-UE relay is authorized to use a specific UE-to-UE service or to serve a specific UE-to-UE service. When the source UE or the target UE or the L3 UE-to-UE relay registers in the 3GPP network and is authorized to use the UE-to-UE service, the network provides a token stating what kind of UE-to-UE service it can use or serve. The token has an expiration time and is signed with a private key. The network also provides the public key to the UEs to be used for verifying the token from other parties.

Editor’s Note: Which network function provides authorization token and how the UEs get the public key of token signing entity are FFS.

### 6.4.2	Solution details

Figure 6.4.2-1 illustrates the high-level procedure of the proposed solution.

Figure 6.4.2-1: High-level procedure of PC5 security between Source/Target UE and UE-to-UE relay

0. 	The 5G ProSe Source/Target UE and UE-to-UE relay are provisioned with the discovery security materials and PC5 security policies, and request authorization tokens when they are in coverage.

1.	The discovery procedure for UE-to-UE Relay is performed by the 5G ProSe Source UE using the discovery parameters and discovery security material, based on the Relay Service Code for UE-to-UE Relay.

2.	If discovery result indicates the UE-to-UE Relay supports Direct Relay service authentication and authorization, the 5G ProSe Source UE sends a Direct Communication Request (DCR) that contains Relay Service Code (RSC) of the 5G ProSe UE-to-UE Relay service and Authorization token of 5G ProSe Source UE which is retrieved from step 0, and also the Key_Est_Info used for direct authentication and key establishment.

Editor’s Note: Its FFS whether the included parameters in Direct Communication Request message in step 2 can be protected by the discovery keys similar as to the Direct Communication Request message when establishing a secure PC5 link in between Remote UE and UE-to-network relay.

Editor’s Note: The need for authorization token is FFS”.

Editor’s Note: Its FFS how token freshness is guaranteed as any peer UE can replay the token (since sent in the clear in DCR).

3.	Direct Auth and Key Establish procedure as specified in TS 33.536 [9] is performed.

4.	The 5G ProSe UE-to-UE Relay uses the public key provided by the network to verify the token of the 5G ProSe Source UE that the 5G ProSe Source UE is authorized to get the UE-to-UE relay service.

5.	The 5G ProSe UE-to-UE Relay derives KNRP and other security material as specified in TS 33.536 [9]. The 5G ProSe UE-to-UE Relay sends a Direct Security Mode Command message to the 5G ProSe Source UE including the Authorization token of 5G ProSe UE-to-UE Relay which is retrieved from step 0.

6.	The 5G ProSe Source UE uses the public key provided by the network to verify the token of the 5G ProSe UE-to-UE Relay that the 5G ProSe UE-to-UE Relay is authorized to provide the UE-to-UE relay service. The 5G ProSe Source UE derives KNRP and other security material similar as the 5G ProSe UE-to-UE Relay in step5.

7.	The 5G ProSe Source UE sends the Direct Security Mode Complete message to the 5G ProSe UE-to-UE.

8.	The 5G ProSe UE-to-UE Relay responds a Direct Communication Accept message to the 5G ProSe Source UE to complete the PC5 connection establishment procedure.

9.	Step 1-8 are repeated for PC5 security establishment between the 5G ProSe Target UE and 5G ProSe UE-to-UE Relay.

10. 	The 5G ProSe Source UE and the 5G ProSe Target UE may establish an end-to-end Security via 5G ProSe UE-to-UE relay. The detail is not described in this solution.

Editor’s Note: The method for providing End to End IP security is FFS.

Editor’s Note: The need of End-to-end security in L3 relay is FFS,

Editor’s Note: The impact on the protocol stack to support end-to-end security for a L3 relay is FFS.

**** END OF CHANGE ****