3GPP TSG-SA3 Meeting #108e 	draft-S3-221805-r1

- e-meeting, 22 - 26 August 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	New KI related to authorization of exposed PIN capabilities

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Please add to FS_PIN a new key issue about authorization of exposed PIN capabilities

# 2	References

[1]	3GPP TR 33.882: "Study on personal IoT networks security aspects."

[2]	3GPP TR 23.700-88: "Study on Personal IoT Networks"

[3]	3GPP TS 33.501. "Security architecture and procedures for 5G system."

# 3	Rationale

As part of the architecture for personal IoT networks currently defined in [2] application functions might need to configure aspects of a PIN using the Network Exposure Interface. SA3 should study in the accompanying study [2], whether there are any security issues related to this and how these can be solved. A corresponding new key issues is proposed in this contribution.

# 4	Detailed proposal

Pleased apply the following changes to [1].

**** START OF 1st CHANGE ****

## 5.Y	Key Issue #Y: Authorization of PIN capabilities

### 5.Y.1	Key issue details

Some aspects of a PIN network might be configurable by an Application Function through the 5G NEF, for instance (depending on details defined in the SA3 study on PIN [2]) QoS of a PIN Element or URSP rules related to a PIN Element.

From a security point of view the scope of access granted to an AF needs to be restricted to the level of certain PEGSs or PINs and needs to be subject to permisssions and consent granted by resource owners.

So far TS 33.501 [y] defines authorization of exposure capabilities on a rather general level in Clause 12. That is, authorization is based on operator policies using the identity of the AF (clause 12.2 in [y]) as well as the OAuth authorization mechanism (Clause 12.4 in [y]). No details about handling of permissions or providing consent to a specific application function are defined.

In case of PIN the requirements for API security might be especially demanding, since on the one hand a PIN network might consist of several UEs and on the other hand a single UE might contribute to several PINs.

Therefore, aspects related to ownership and possible operation models of PINs shall be included in the analysis of the Key issue.

### 5.Y.2	Security threats

An application function associated with one PIN might use the NEF API to manipulate another PIN.

An application function associated with a PIN might use the NEF API to manipulate resources not assigned to the PIN.

### 5.1.3	Potential security requirements

The 5GS shall be able to restrict resource request from an Application Fuction associated with a PIN to the resources associated with the PIN.

Application functions associated with a PIN shall be able to use APIs for accessing resource only with authorization from the resource owner..

**** END OF 1st CHANGE ****

**** START OF 2nd CHANGE ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-88: "Study on Personal IoT Networks"

[y]	3GPP TS 33.501. "Security architecture and procedures for 5G system."

**** END OF 2nd CHANGE ****

**** START OF 3rd CHANGE ****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TR 23.700-88 [2], and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF		Application Function

API	Application Programming Interface

NEF	Network Exposure Function

PEGC	PIN Element with gateway capabilities

PIN	Personal IoT Network

PINE	PIN Element

QoS	Quality of Service

URSP	UE Route Selection Policy

<ABBREVIATION>	<Expansion>

**** END OF 3rd CHANGE ****