3GPP TSG-SA3 Meeting #108e 	draft-S3-221931-r4

- e-meeting, 22 - 26 August 2022

- Source:	Ericsson, Lenovo, Nokia, Nokia Shanghai Bell, Rakuten Mobile Inc., Interdigital, US NSA, Motorola Solutions, Johns Hopkins University APL, Intel, Center for Internet Security, China Mobile, ZTE, CableLabs

- Title:	Key Issue#1 Need for trust evidence collection for continuous trust monitoring and evaluation

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

This pCR adds a key issue to TR 33.894.

# 2	References

[1]	SP-220674, ‘New Study on applicability of the Zero Trust Security principles in mobile networks’, Rel.18.

[2] NIST Special Publication 800-207, ‘Zero Trust Architecture’, August 2020.

# 3	Rationale

The tenets of Zero Trust include continuous monitoring of integrity and security posture of all owned and associated assets, network infrastructure, and communications [2, tenets 4, 5, 7]. Continuous trust monitoring is one of the key enablers for providing fine-grained access control decisions on a per-session basis to minimize impacts if any security incidents occur. To perform continuous trust monitoring, there is a need for data, that allows for observation of the security status of 5GC NFs and making decisions based on such monitoring. Currently, there is no standardised feature in 5GS that can provide trust evidence data that would supply continuous trust monitoring of 5GC NFs. Therefore, this pCR proposes a key issue on introducing key components to support continuous trust monitoring of 5GC NFs.

S3-221956 (KI#1 Need for continuous trust evaluation) and S3-221958 (New Key Issue on Usage of Evaluated Trust Information) are merged in S3-221931.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.894.

*****Start of Change 1*****

## 5.X	Key Issue #1: Need for trust evidence collection for continuous trust monitoring and evaluation/monitorn

### 5.X.1	Key issue details

The 5G system includes heterogenous and varied network functions (NF) deployments which may face issues such as the NF may run into errors due to configuration issues, the NF may get exposed to insider threats or cyber attacks etc., Once a NF is compromised or under attack, the services offered by the NF may also be impacted. Lateral movement of the threat to other connected NF can further increase the threat surface. So, the trust over any NF once deployed may not be assumed static and intact throughout its lifetime despite various existing security pre-configurations (such as to support authentication or authorization etc.,). Various lifecycle events (apart from static configuration at deployment) may impact the trust level (e.g., X.509 certificates lifecycle events such as expiry, renewal and revocation, abnormal NF behaviour, etc.).

Currently, there is no standardised feature in 5GS that can automatically collect and provide continuous dynami monitoing and evaluation/monitoring wihsary data uld have procedures for gathering and preparing necessary data to be collected by a trust monitoring and evaluation/monitoring mechanism. On realization of the trus  and evaluation of a target NF, any associated and authorized NF can make use of the information to apply necessary security controls such as utilization and access control for various scenarios (e.g., access control associated to service authorization, service access, network function selection, usage etc.,). Secure collection, transfer, and storing of ta assumes the itinterfaces for 5GC NFsropriate access c and integrity protecti insit

There are multiple use cases where trust evaluation may be required and brings benefits within an operator network as well as across multiple operators’ networks. The continuous trust monitoring and evaluationmg is need to apply appropriate security controls based on operator policies to detect and mitigate any anomalies.

### 5.X.2	Security threats

Based on ZT principles, assume that NF is constantly under threat or compromised in the core network, lack of data for continuous trust monitoring and evaluationmwill allow the impacted NF(s) to remain unidentified in the network and involved in various network services, which may lead to a service failure, data loss/theft, or lateral movement of the attack.

### 5.X.3	Potential security requirements

The 5GC shall support automatic and secure collection, transfer, and storing of the data for  continous trust monitoring and evaluation/monitoi

The 5GS shall support mechanisms to provide the trust monitored and evaluated information about a NF to the authorized consumers.

NOTE 1: The algorithms or logic for trust  and the outcome of ttion/monitoring are outside the scope of 3GPP.

*****End of Change 1*****