3GPP TSG-SA3 Meeting #108-e 	S3-221829

- e-meeting, 22 - 26 August 2022

- Source:	Qualcomm Incorporated

- Title:	Re-using V2X unicast security for UAS

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This contribution proposes a security solution for key issues #1 and #2 re-using the V2X security.

# 2	References

[1]	3GPP TR 33.891 v0.1.0 (S3-221067)

[2]	3GPP TS 33.536

# 3	Rationale

This contribution proposes to re-use the procedures for establishing unicast security for V2X services given in TS 33.536 [2] for UAS.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.570-58: " Study of further architecture enhancements for uncrewed aerial systems and urban air mobility".

[3]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP".

[4]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking".

[5]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[aa]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

**** NEXT CHANGE ****

- 6.X	Solution #X: C2 and DAA unicast security using V2X unicast solution

- 6.X.1	Introduction

This solution addresses both key issue #1 and #2. It re-uses the security procedures for unicast security for V2X (see TS 33.536 [aa]).

- 6.X.2	Solution details

TS 33.536 [aa] contains a method of establishing security for a unicast connection for V2X services. This solution proposes to re-use those procedures for establishing the security of C2 and DAA unicast connections. The relevant procedures are given in clause 5.3.3 of TS 33.356 [aa].

Editor’s note: Whether there is a need for discovery before communication will be base on an SA2 decision.

Editor’s note: More details on the credentials for establishing DAA connections are FFS.

Editor’s note: Details on authorisation for C2 pairing are FFS.

Figure 6.X.2-1: Message flow for the establishment of unicast security

Figure 6.X.2-1 gives an overview of the flow for (re-)establishing security for unicast connections (see TS 33.536 [aa] for the details). The security policy relevant to the service (i.e. DAA or C2) will be used when establishing a connection for that service.  C2 and DAA traffic will only be accepted from a peer authorised to send such traffic.

Editor’s note: The need for and security issues of DAA and C2 traffic sharing a connection are FFS.

6.X.3	Evaluation

The evaluation is FFS.

**** END OF CHANGES ****