3GPP TSG-SA3 Meeting #108-e 	S3-222053r3

- e-meeting, 22 - 26 August 2022

- Source:	OPPO

- Title:	New KI on U2U relay protection of remote UE traffic

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes a new KI on U2U relay protection of remote UE traffic.

# 2	References

[1]	TR 33.740

# 3	Rationale

During SA3-107bis-e meeting, it was agreed that the U2U relay is trusted. The security between two remote UEs connected via a U2U relay can be either realized end-to-end, hop-by-hop, or both .

Any one of the three protection schemes can satisfy the potential security requirements in KI#2:

The 3GPP system shall support a means to provide confidentiality, integrity and replay protection of end-to-end information exchanged between the peer UEs over the UE-to-UE Relay.

The 3GPP system shall support a means to protect security (i.e., the integrity, confidentiality, and replay protection) of user-plane and control-plane messages, including during UE-to-UE Relay path switch.

However, from an efficiency point of view, it does not make sense to do both end-to-end and hop-by-hop at the same time. It is proposed to add a KI that deals with negotiating a protection scheme among U2U relay and remote UEs so that everyone is on the same page.

# 4	Detailed proposals

/************************* BEGIN CHANGES***************************************/

- 5.X	Key Issue #X: Security of source and target UE communication via U2U relay

- 5.X.1	Key issue details

3GPP system has to be able to protect security (i.e., the integrity and confidentiality) of information between the source and target UEs over the UE-to-UE Relay. With the U2U relay being trusted, the protection of security between source and target UEs connected via the U2U relay can be realized either hop-by-hop or end-to-end or both.  However, from an efficiency point of view, it does not make sense to do both hop-by-hop and end-to-end at the same time, especially since the U2U relay scenario most likely focuses on the case where the UEs are out of network coverage and therefore out of power source.

This key issue addresses the need for the U2U relay and the source and target UEs to negotiate and agree on a protection scheme, whether it is hop-by-hop or end-to-end. It is independent of the need to apply integrity-, confidentiality-, and replay-protection once the protection scheme is negotiated. Key issue is for L3 relay.

- 5.X.2	Security threats

If remote UEs communicating with each other via U2U relay are not using the same protection scheme (i.e., hop-by-hop or end-to-end), the source and target UEs will not be able to communicate with each other or the traffic between the source and target UEs may not be protected at all.

- 5.X.3	Potential security requirements

The 3GPP system shall support a means for the source UE, target UE and U2U relay to use the same.ecurity protection scheme in protecting UE-to-UE traffic.

/************************* END CHANGES***************************************/