3GPP TSG-SA3 Meeting #108e 	draft_S3-222137-r1

- e-meeting, 22 - 26 August 2022

- Source:	China Mobile

- Title:	Adding a test case of AKMA key strorage and update

- Document for:	Approval

Agenda Item:	4.5

# 1	Decision/action requested

This document proposes to add a test case of AKMA key strorage and update in TS 33.537.

# 2	References

[1]	3GPP TS 33.537 Security Assurance Specification (SCAS) for the Authentication and Key Management for Applications (AKMA) Anchor Function (AAnF) v0.1.0

# 3	Rationale

This document proposes to add a test case of AKMA key strorage and update in TS 33.537 [1].

# 4	Detailed proposal

*** START CHANGES ***

## 4.2	AAnF-specific adaptations of security functional requirements and related test cases

### 4.2.1	Introduction

The present clause contains AAnF-specific security functional adaptations of requirements and related test cases.

### 4.2.2	Security functional requirements on the AAnF deriving from 3GPP specifications and related test cases

#### 4.2.2.1	AKMA key storage and update

Requirement Name: AKMA key storage update

Requirement Reference: TS 33.535 [4], clause 6.1

Requirement Description: “The AAnF shall store the latest information sent by the AUSF. After receiving the new generated A-KID and KAKMA, the AAnF deletes the old A-KID and KAKMA and stores the new generated A-KID and KAKMA.” as specified in TS 33.535[4], clause 6.1.

Threat References: TR 33.926 [6], clause X.2.2.Y, AKMA key storage and update

Test Case:

Test Name: TC_AKMA_Key_Storage_Update

Purpose:

Verify that the AAnF stores the latest AKMA context received by the AUSF, which means only one AKMA context per SUPI is stored in AAnF.

Pre-Conditions:

-	Test environment with AUSF. The AUSF may be simulated.

-	AAnF network product is connected in emulated/real network environment.

Execution Steps

Test A:

1)	The AUSF sends the generated A-KID and KAKMA to the AAnF together with the SUPI of the UE using the Naanf_AKMA_KeyRegistration Request service operation. The A-KID and KAKMA are different from the current ones.

Expected Results:

The tester can only find one A-KID and KAKMA which are different from the the old A-KID and KAKMA in AAnF.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

*** END of CHANGES ***