3GPP TSG-SA3 Meeting #108e 	draft_S3-222207-r5

- e-meeting, 22 - 26 August 2022										            		    Revision of S3-22xxxx

- Source:	Xiaomi, Huawei, Hisilicon, ChinaTelecom

- Title:	33.893: New Key Issue on Security for PC5 Communication

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed new key issue to be added in TR 33.893.

# 2	References

[1]	TR 33.893	Study on Security Aspects of Ranging Based Services and Sidelink Positioning

# 3	Rationale

Based on the current version of TR 33.893 [1], this pCR proposes a new key issue on security for unicast direct communication between the UEs over PC5 link for Ranging/Sidelink Positioning service.

Merger of S3-222084, S3-221916

# 4	Detailed proposal

*************** Start of the Change ****************

# 5	Key issues

5.X	Key issue #X: Protection of direct communication over PC5

### 5.X.1	Key issue details

As per TR 23.700-86 [2], for direct communication between the UEs, the architecture and solutions defined for 5G V2X and 5G ProSe will be reused as much as possible. This provides the basis for reusing the direct communication security defined for 5G ProSe in TS 33.503 [6] or for 5G V2X in TS 33.536 [5] to protect the direct communication for Ranging/SL Positioning services.

Although the security mechanisms for direct communication of 5G ProSe or 5G V2X services can be reused for Ranging/SL Positioning services, there are still some scenarios in Ranging/SL Positioning services that are not discussed and studied for 5G ProSe or 5G V2X services. Considering the Ranging/SL Positioning services may have different processing procedures, it’s necessary to study the security of direct communication which is dedicated to the Ranging/SL Positioning services scenario.

In addition, for almost all Ranging/Sidelink Positioning services, the information exchanged during PC5 direct communication between the UEs is location related, which is security/privacy sensitive. This is also an aspect different from 5G ProSe or 5G V2X services which do not always carry security/privacy sensitive information over PC5.

5.X.2	Security threats

During PC5 direct communication establishment, if the UE cannot authenticate the peer UE to be the entity it intends to communicate with, it may lead to the disclosure of privacy-sensitive information to the peer UE.

Failure to protect the integrity of Ranging/SL Positioning service information during PC5 direct communication will open vulnerability for attacks such as fabrication, modification, or removal of the Ranging results.

Failure to protect the confidentiality of Ranging/SL Positioning service information during PC5 direct communication will open vulnerability for eavesdropping attacks resulting in privacy violations.

In case one UE is communicating with multiple peer UEs for the same Ranging/SL Positioning service, if there is no security isolation between the PC5 direct links with multiple peer UEs, one compromised peer UE or compromised PC5 link could lead to the compromise of all PC5 links with other peer UEs.

5.X.3	Potential security requirements

The 5G system shall support a means for the Ranging-capable UEs to mutually authenticate each other during PC5 direct communication of Ranging/SL Positioning service.

The 5G system shall support a means to protect the integrity of the information transferred during PC5 direct communication for the Ranging/SL Positioning service.

The 5G system shall support a means to protect the confidentiality of the information transferred during PC5 direct communication for the Ranging/SL Positioning service.

The 5G system shall ensure that PC5 security policies are properly set and enforced to activate the required security during PC5 direct communication for the Ranging/SL Positioning service.

The 5G system shall support a means for the Ranging-capable UE to establish a different security context for each peer UE during the PC5 direct communication establishment of Ranging/SL Positioning service.

*************** End of the Change ****************