3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231931

# Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Updating the procedure to align with the R-17 agreement

# Document for:	Approval

Agenda Item:	4.14

1	Decision/action requested

Approve the changes in clause 4 for inclusion in the living CR of PROSESA.

2	References

[1]	S3-231620	SA3#110

3	Rationale

The changes are based on the living CR approved as S3-231620.

The basic procedure should be updated to align with TS 23.403 and the TS 33.503vh.3.0.

4	Detailed proposal

************ START OF CHANGES ************

6.3.3.3.x.2	PDU Session secondary authentication of 5G ProSe Remote UE via 5G ProSe Layer-3 UE-to-Network Relay

The PDU session secondary authentication of 5G ProSe Remote UE via 5G ProSe Layer-3 UE-to-Network Relay follows the steps described in figure 6.3.3.3.x.2-1.

Figure 6.3.3.3.x.2-1: Procedure for PDU session secondary authentication of 5G ProSe Remote UE 
via 5G ProSe Layer-3 UE-to-Network Relay

0.	During the Registration procedure, authorization and provisioning are performed for 5G ProSe Remote UE(0a) and 5G ProSe Layer-3 UE-to-Network Relay(0b) as described in clause 5.1.4 of TS 23.304 [2].

1.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU session for relaying with default PDU session parameters as described in clause 6.5.1.1 in TS 23.304 [2].

2.	Based on the authorization and provisioning in step 0, the 5G ProSe Remote UE performs the discovery of a 5G ProSe Layer-3 UE-to-Network Relay. As part of the discovery procedure, the 5G ProSe Remote UE learns about the connectivity service the 5G ProSe Layer-3 UE-to-Network Relay provides (e.g. based on a broadcasted service code) as described in clause 6.3.1.2 or 6.3.1.3 of TS 23.304 [2].

3.	The 5G ProSe Remote UE selects a 5G ProSe Layer-3 UE-to-Network Relay sends a DCR (Direct Communication Request) message including its SUCI or a 5GPRUK ID as described in clause 6.3.3.3.2.

4.	The Remote UE runs CP or UP based authentication as described in 6.3.3.3.2, or 6.3.3.2.2. In addition, the following procedure may happen in this step as described in clause 6.5.1.1 in TS 23.304 [2].

If there is no PDU session satisfying the requirements of the PC5 connection with the 5G ProSe Remote UE, e.g. S-NSSAI, DNN, QoS, UP security activation status, the 5G ProSe Layer-3 UE-to-Network Relay initiates a new PDU session establishment or modification procedure for relaying.

5.	Upon successful network-controlled authentication of 5G ProSe Remote UE procedure, the 5G ProSe Layer-3 UE-to-Network Relay initiates a Direct Security Mode Command procedure with the 5G ProSe Remote UE as described in clause 6.2.3.

6.	Upon successful security establishment, the 5G ProSe Layer-3 UE-to-Network Relay stores the 5GPRUK ID as described in clause 6.3.3.3.2 and sends a DCA (Direct Communication Accept) message to the Remote UE. The DCA may include an indication that a PDU Session with secondary authentication is pending if the L3 UE-to-Network Relay determines the DN that is associated with the relay service code requires secondary authentication for the 5G ProSe Remote UE based on the fact that the L3 UE-to-Network Relay performed secondary authentication with the same DN either in step 1 or step 4, and there is no stored authentication information associated with the Remote UE. Based on the indication in the DCA message, the 5G ProSe Remote UE may refrain from sending any data traffic over the PC5 link until successful completion of subsequent PDU Session secondary authentication.

7.	For IP PDU Session Type and IP traffic over the PC5 reference point, the IPv6 prefix or IPv4 address is allocated for the 5G ProSe Remote UE as defined in clause 5.5.1.3 in TS 23.304 [2]. In addition, the 5G ProSe Layer-3 UE-to-Network Relay may configure a traffic filter (e.g. as a default filter for IP or non-IP traffic) for the PC5 link to prevent any data traffic until successful completion of subsequent PDU Session secondary authentication.

8.	The 5G ProSe Layer-3 UE-to-Network Relay sends a Remote UE Report message to the SMF for the PDU session associated with the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay shall include the 5GPRUK ID as the Remote User ID and 5G ProSe Remote UE addressing info (e.g. IP or MAC address). The Remote UE Report message includes the 5G ProSe Remote UE info (Remote User ID, addressing info) and excludes other 5G ProSe Remote UEs info. The Relay shall additionally include the 5GPRUK ID in the subsequent NAS messages. The SMF shall use the 5G PRUK ID to get SUPI as described in clause 6.3.3.

Editor's Notes: How to support multiple Remote User IDs in Remote UE Report is FFS.

NOTE 1:	In the case of Home Routed roaming, the SMF in the call flow is the H-SMF (and the V-SMF is not shown for simplicity). SMF selection by AMF is performed as per TS 23.502 [13], clause 4.3.2.2.3 (e.g. using PLMN ID of the SUPI, S-NSSAI, etc.).

9.	When the SMF receives Remote UE Report the SMF retrieves Remote UE's SM subscription data from the UDM by triggering Nudm_SDM_Get service operation. The SMF may include DNN, S-NSSAI of the PDU Session for relaying in addition to the Remote UE's SUPI as input parameters. The SMF determines based on the subscription data of the 5G ProSe Remote UE (i.e. Secondary authentication indication as per TS 23.502 [13], Table 5.2.3.3.1). The SMF may also check whether the 5G ProSe Remote UE has been authenticated by the same DN as indicated in the subscription data and, if secondary authentication is required, the SMF triggers a PDU Session secondary authentication of 5G ProSe Remote UE via 5G ProSe Layer-3 UE-to-Network Relay by sending PDU Session Authentication Command message to the 5G ProSe Layer-3 UE-to-Network Relay including the 5GPRUK ID of the Remote UE and an EAP-Request/Identity.

Editor's Notes: how SMF is notified with the 5G ProSe remote UE's subscription update is FFS.

Note 2:	The information on a successful authentication between a 5G ProSe Remote UE and an SMF may be saved in SMF and/or UDM.

10.	Based on the 5GPRUK ID, the 5G ProSe Layer-3 UE-to-Network Relay forwards the EAP-Request/Identity to the 5G ProSe Remote UE via PC5 signalling(10a). The 5G ProSe Remote UE returns the EAP-Response/Identity to the 5G ProSe Layer-3 UE-to-Network Relay via PC5 signalling(10b).

11.	The 5G ProSe Layer-3 UE-to-Network Relay sends PDU Session Authentication Complete message to the SMF including the 5GPRUK ID of the Remote UE and an EAP-Response/Identity received from the 5G ProSe Remote UE.

12.	The SMF sends an EAP-Response/Identity to the DN-AAA.

13.	The DN AAA server and the UE should exchange EAP messages, as required by the EAP method. The SMF and Relay shall include the 5GPRUK ID in the NAS messages transporting the EAP messages.

14.	The DN-AAA sends EAP-Success or EAP-Failure to the SMF.

15.	Upon successful PDU Session secondary authentication via the Relay procedure, the SMF stores the 5G ProSe Remote UE information in the 5G ProSe Layer-3 UE-to-Network Relay's SM context including 5G ProSe Remote UE identity (e.g. GPSI, SUPI), individual authentication information received from DN-AAA.

16.	The SMF sends Remote UE Report Ack message to the 5G ProSe Layer-3 UE-to-Network Relay indicating the result of the PDU Session secondary authentication, including the 5GPRUK ID of the remote UE and an EAP success or failure message. In the case of successful secondary authentication, the message may include QoS authorization info for the 5G ProSe Layer-3 UE-to-Network Relay to enforce. In case the secondary authentication is failed, the NAS message may indicate that 5G ProSe Layer-3 UE-to-Network Relay should release the PC5 link with the 5G ProSe Remote UE.

17.	In the case of successful secondary authentication for the 5G ProSe Remote UE, the 5G ProSe Layer-3 UE-to-Network Relay stores any received authentication info associated with the 5G ProSe Remote UE. In case the secondary authentication is failed, the 5G ProSe UE-to-Network Relay releases the PC5 link with the 5G ProSe Remote UE and may keep the PDU session as the default PDU session or release it if there is no more 5G ProSe Remote UE using the same PDU session.

************ END OF CHANGES************