3GPP TSG-SA3 Meeting #110Ad-Hoc-e	S3-232086

# Electronic meeting, Online, 17 - 21 April 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	evaluation for KI#2 temporary slice authorization in NSSAA

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve the evaluation to TR33.886

# 2	References

[1]	3GPP TR 23.700-41 “Study on enhancement of network slicing; Phase 3”

[2]	3GPP TR 33.886 "Study on enhanced security for Network Slicing Phase 3"

# 3	Rationale

The contribution proposes evaluation of solution for KI#2 to solve potential issue on network slice specific authentication and authorization caused by temporary slice. All content in the change part is new.

# 4	Detailed proposal

***************  Start of 1st change  ************

## 5.X	Solution #X: temporary slice authorization in NSSAA

### 5.X.3	Evaluation

This solution addresses KI#2 to solve potential issue on network slice specific authentication and authorization caused by "garbage" produced by temporary slices during NSSAA procedure.

The solution proposed to enhance NSSAA procedure to add validity timer of a temporary slice in NSSAA authentication request as optional parameter. If it's existed, AAA-S triggers NSSAA revocation procedure when detect timeout of the temporary slice and may include a slice timeout indication in the revoke auth request. After received revoke auth request with the slice timeout indication, NSSAAF cleans-up local status related to the temporary slice, then sends response to AAA-S without further notifying AMF, as it assumed the AMF and UE will cleanup context of the temporary slice locally.

Impacts on existing entities and interfaces:

AMF:

- support an optional IE, e.g., validity timer, in NSSAA authentication request.

NSSAAF:

- support an optional IE, e.g., validity timer, in NSSAA authentication request and AAA protocol message.

- support an optional IE, e.g., slice timeout indication, in AAA protocol revoke auth message.

NOTE: how does NSSAAF cleanup the local context for the temporary slice is implementation dependant.

NOTE: how does AAA-S detect the timeout of a temporary slice and trigger revocation is out of scope of 3GPP.

gNB: None

UE: None

*************    End of the changes	*********