# 3GPP TSG-SA3 Meeting #112 	S3-233719

# Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Authorization for UE role in Ranging

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the changes in TS 33.533.

2	References

[1]	TR 33.893

3	Rationale

Based on the conclusion on Key Issue #2 in TR 33.893, the procedure for authorization of UE role is added.

4	Detailed proposal

************ START OF CHANGES ************

6.3.X	Procedure for authorization of UE for Ranging/SL positioning service exposure

For UE role authorization, the UE role info shall be included in the request message towards the peer UE. The authorization of UE role is performed by the peer UE via checking the locally pre-configured authorization or privacy verification information, which is provisioned either from the application or from the network. If the UE role is not acceptable, the peer UE shall reject the request directly or response with the preferred UR role.

************ END OF CHANGES************