3GPP TSG-SA3 Meeting #112 	S3-234050

Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Lenovo, BROADCOM CORPORATION, CableLabs, CATT, Charter Communications, Inc, China Mobile, CISCO, Deutsche Telekom, InterDigital, Inc., LG Electronics, Nokia, Tencent, vivo Mobile Communication Co.,, Xiaomi, ZTE Corporation

# Title:	New SID on QUIC optimization for access traffic steering, switching and splitting support in the 5G system architecture; Phase 3

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New SID on QUIC optimization for access traffic steering, switching and splitting support in the 5G system architecture; Phase 3

Acronym:	ATSSS_Ph3_Sec

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ATSSS_Ph3 | SA2 | 940070 | Study on Access Traffic Steering, Switching and Splitting support in the 5G system architecture; Phase 3 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

# 3	Justification

SA2 is defining in ATSSS the use of Multipath QUIC (https://datatracker.ietf.org/doc/draft-ietf-quic-multipath) between the UE and the UPF in the selected solution #2.2 in TR 23.700-53. MP-QUIC extends QUIC (RFC 9000) to enable the simultaneous use of multiple paths for a single connection. QUIC requires according to RFC 9001 the mandatory usage of TLS 1.3 with encryption according to RFC 8446. This mandatory requirement is reasonable for the intended use of QUIC in the normal client – application server environment in the internet. In the SA2 discussion, the double encryption on QUIC layer and on the access layer was seen as computational burden in the UPF and there was a desire to have an optional encryption/integrity protection/authentication on QUIC layer in case access layer security is used.

SA2 captured the following Note in the conclusions of KI#2 accordingly:

NOTE 2:	SA WG3 can study security optimizations that can improve the user-plane performance, such as whether the encryption in the QUIC layer can be omitted.

This SID intends to study if encryption/integrity protection/authentication of QUIC between UE and UPF can be omitted, and to study the mechanisms to do so for MPQUIC for ATSSS.

The outcome of the study should be applicable also for applications other than QUIC and incorporate feedback from IETF.

# 4	Objective

The objective is to study

- whether the encryption/integrity protection/authentication at the QUIC layer of MPQUIC between UE/UPF can be omitted, and

- whether to specify mechanisms to omit encryption/integrity protection/authentication at QUIC layer in order to optimize the performance of MPQUIC for ATSSS.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | N/A | TLS profile for QUIC with NULL encryption/integrity protection | SA#104 | SA#105 | Andreas Kunz |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A |  |

# 6	Work item Rapporteur(s)

Andreas Kunz (akunz@lenovo.com)

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

System architecture aspects, i.e. QUIC procedures, will be covered by SA2

# 9	Supporting Individual Members

Supporting IM name

Lenovo

BROADCOM CORPORATION

CableLabs

CATT

Charter Communications, Inc

China Mobile

CISCO

Deutsche Telekom

InterDigital, Inc.

LG Electronics

Nokia

Tencent

vivo Mobile Communication Co.,

Xiaomi

ZTE Corporation